Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ACOUSTICS AND OPTICS							               Time : 3 hours

Subject Code: 	PH201			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The image formed by a plane mirror is _____________.

2.	In the case of plane wavefront, the source is placed at __________.

3.	What is meant by magnification of a lens?

4.	Spherical aberration can be removed by using ____________.

5.	State the properties of laser beam.

6.	What is meant by multimode fiber?

7.	The phenomenon of waxing and waning of sound is called ___________.

8.	What is the percentage change in intensity represented by one decibel?

9.	The persistence of sound after the source has stopped is known as __________.

10.	The unit for absorption coefficient is _____________.

PART – B  (5 x 3 = 15 MARKS)

11.	Prove that when a plane mirror is rotated through an angle , the reflected light rotates through double the angle 2.

12.	Explain power of lens and its unit.

13.	Explain the principle used in CD – ROM.

14.	Give the characteristics of wave motion.

15.	Explain transmission loss of a sound energy.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Describe Michelson’s interferometer and explain the formation of fringes in it.		(12)

b.	A shift of 100 fringes is observed when the movable mirror of the Michelson interferometer

is shifted by 0.0295 mm. Calculate the wavelength of light.						(3)

(OR)

17.	a.	Discuss in detail the production, behaviour, and analysis of plane, circular and elliptically

polarized light.																(12)

b.	Give the three uses of Polaroid’s.												(3)

18.	a.	Explain with suitable figures, the longitudinal spherical aberration and lateral spherical 			aberration.																	(10)

b.	A double converse lens has radii of curvature of 40 cm and 10 cm. Find the longitudinal chromatic observation for an object at infinity. Given  = 1.523 and R = 1.5145.	(5)

(OR)

19.	a.	Discuss in detail the construction and working of Lummer-Brodhun photo meter and explain

how the illuminating power of two sources can be compared.						(12)

b.	In a hall a bulb is lighted at a height of 5 metre from the floor. Compare the illumination

density at a point P on the floor just below the bulb and at a point Q on the floor at a distance 12 metre from P.														(3)

[P.T.O]

20.	Discuss in detail the construction and working of He-Ne laser with suitable energy level diagram

and also give its applications.

(OR)

21.	a.	Derive the expressions for acceptance angle and numerical aperture of an

optical fiber.																	(8)

b.	Explain the construction and reconstruction of a hologram with their conditions.		(7)

22.	Derive an expression for the velocity of sound in a gas. Discuss the effect of pressure, temperature and humidity of the gas on the velocity.

(OR)

23.	Describe the formation of stationary waves in open and closed pipes. Explain how nodes and antinodes are formed.

24.	What is reverberation time and derive sabines formula for the same.

(OR)

25.	a.	Discuss 	in detail the factors affecting the architectural acoustic and also give

their remedy.																	(12)

b.	Give the characteristics of a good absorbing materials.							(3)