Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	UNIT OPERATIONS										Time : 3 hours

Subject Code:	CL222		         						               	 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Steady state heat transfer occurs, when the flow of heat is _______________.

2.	Energy can be converted from one form to another. This is _______________.

3.	Ratio of kinematic viscosity to thermal diffusivity is known as _____________.

4.	Higher the temperature of the radiating body _______ is the wave length of radiation.

5.	Multi pass heat exchanger is used to ___________________.

6.	Heat transfer coefficient is the quotient of ______________________________.

7.	The separation of a suspension into a supernatant clear fluid and a rather dense slurry containing 	a higher concentration of solid is called ________________.

8.	Filter used for producing a thick suspension from a thin slurry is ________________.

9.	Highly viscous liquids and pastes are agitated by _________________.

10.	Kinetic similarity is obtained when there is equal _________________.

PART – B  (5 x 3 = 15 MARKS)

11.	What is the general form of Fourier’s law of heat conduction through a cube?

12.	Calculate the heat transfer coefficient for water at 60oC, flowing through a tube of ID 0.625 cm with a velocity of 0.9 m/sec. Tube wall temperature is 40oC. At the mean temperature,           µwater = 2.177 kg/mhr, K = 0.54 kcal/mhroC, Cp = 1 kcal/kgK and ρ = 1000 kg/m3.

13.	What is Duhring’s rule and what is its use in evaporation?

14.	Explain what you mean by flotation and froth flotation.

15.	How would you prevent swirling in agitated vessel?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Derive an expression for steady state heat transfer by conduction through three multiple

cylindrical layers. 							        						      (10)

b.	A furnace wall consists of three layers. The inner layer of 10 cm thickness is made of

firebrick (K=1.04 W/mK). The intermediate layer of 25 cm thickness is made of masonry brick (K=0.69 W/mK) followed by a 5 cm thick concrete wall (K=1.37 W/mK). Even the furnace is in continuous operation, the inner surface of the furnace is at 800oC while the outer concrete surface is at 50oC. Calculate the rate of heat loss per unit area of the wall. 	 																			 (5)

(OR)

17.	a.	Derive an expression for the conduction of heat through a hollow sphere of homogeneous

material with constant inside and outside temperatures. 		   	 (8)

b.	A hot flue gas at 80oC is flowing through a composite spherical shell formed by two spheres having inner, middle and outer radius as; 10cm; 15cm and 20 cm.

Thermal conductivity of inner shell = 38 W/moC

Thermal conductivity of outer shell = 45 W/moC

Film coefficient of heat transfer at the inner surface of composite spherical shell  = 25 W/m2 oC

Film coefficient of heat transfer at the outer surface of composite spherical shell = 10 W/m2 oC

[P.T.O]

Ambient air temperature = 28oC. Calculate:

i.	The values of individual resistances in the composite spherical shell

ii.	The rate of heat flow through the wall of composite spherical shell

iii.	The temperature at the inner surface, at the interface and at the outer surface.	 (7)

18.	Using dimensional analysis derive an expression for forced convection heat transfer under turbulent flow conditions. Also give the physical significance of the dimensionless groups.

(OR)

19.	a.	Write short notes on “Radiation error in temperature measurement”		              		(3)

b.	Derive an expression for heat exchange by radiation between the two nonblack bodies.   (12)

20.	a.	Derive an expression for Log Mean Temperature Difference for a parallel flow double pipe

heat exchanger. 								 	           						   (10)

b.	It is desired to cool oil from 120oC to 50oC using a double-pipe heat exchanger. The cooling water enters the heat exchanger at 20oC and leaves at 40oC. Calculate the LMTD for (i) a parallel flow heat exchanger and (ii) for a counter-current heat exchanger.	   	 		(5)

(OR)

21.	a.	Explain the different methods of feeding multiple effect evaporators with neat sketches.

Explain the salient features of each. 				                                                     (10)

b.	Explain briefly the liquid characteristics which affect evaporation.      		                  (5)

22.	 Describe the various types of Separation processes with examples.

(OR)

23.	 Explain in detail about the following Centrifugal filters

(a) Suspended Batch Centrifuges						 							(8)

(b) Automatic Batch Centrifuges 												       (7)

24.	a.	 Briefly explain the flow patterns in agitated vessels.		         		             			(9)

b.	Name any two mixing equipments used for mixing solid particles or Powders. 	 		(2)

c.	Write short notes on “Geometric and Dynamic Similarities”.	               	   		       (4)

(OR)

25.	Explain the various types of impellers used for Agitation