Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Trimester Examination – September / October 2009

Subject Title: 	FUNDAMENTALS OF CIVIL ENGINEERING		          Time : 3 hours

Subject Code: 	CE382			         						           Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	What is the purpose of surveying?  What are the advantages and disadvantages of chain

surveying?																	( 8 )

b.	What are the temporary adjustments to be done in a levelling instrument?    				( 8 )

c. 	The staff readings taken continuously in a fly levelling from bench mark P(RL = 100 m)

to station Q are the following : 0.835, 1.355, 2.105, 1.475, 2.635, 2.025, 1.920 and 1.945. Find the R.L. of Q. 																	( 4 )

2.	a.	Explain : Spillway and drainage gallery in a dam	.									( 8 )

b.	Draw the cross section of a gravity dam and explain the salient features.					( 8 )

c.	Define : Irrigation. What are the different types of irrigation?							( 4 )

(OR)

3.	a.	Explain : Clearway, Hangar, Landing Strip, Stopway	.								( 8 )

b.	What is an apron? How is it classified?												( 8 )

c.	Explain : Full reservoir Level, Maximum Water Level.									( 4 )

4.	a.	What is an Oxidation Pond? Explain the constructional details of an oxidation pond. 		( 8 )

b.	Explain the principle of operation of a Rapid Sand filter.  								( 8 )

c.	What is the necessity for Sewage treatment and disposal? 								( 4 )

(OR)

5.	a.	Explain the common water treatment arrangement with a general layout.					( 8 )

b.	Draw the cross sectional view of a Septic Tank and explain.							( 8 )

c.	What is the necessity for protected water supply? 									( 4 )

6.	a.	Draw the section of a bituminous road and explain the construction sequence.				( 8 )

b.	Define : Permanent Way. Draw the cross section of a permanent way.					( 8 )

c.	What are the various component parts of a bridge? Explain with sketches.				( 4 )

(OR)

7.	a.	Explain a T beam and Slab Bridge with sketches. To what span is it suitable for?			( 8 )

b.	What do you mean by water bound macadam road.  Explain the cross section of water

bound macadam road with sketches	.											( 8 )

c.	Explain: Carriageway of a bridge.													( 4 )

8.	a.	What is a Pile Foundation? How is pile foundation classified?   Explain with suitable sketches.																					(12)

b.	What are the responsibilities & duties of a Civil Engineer?								( 8 )

(OR)

9.	a.	Explain the different elements in a residential building and its functions.					(12)

b.	What are the functions of a foundation?											( 8 )