Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

M.B.A. Trimester Examination – April/May 2010

Subject Title: 	LEAN MANUFACTURING							             Time : 3 hours

Subject Code: 	09MS344		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Case Study: (Compulsory)

The Rubco rubber manufacturing is a midsize manufacturer of rubber components. The company has seen a dramatic increase in sales due to the increased worldwide demand for the products. The company has recorded growth in the demand in the past few years and is growing at a steady rate. Although the company is achieving higher sales, their profit margins are decreasing and product lead-times are increasing. Though the company is growing, by working harder it is making very less profit. Today, the increase in customer orders has turned the company into chaos; production workers scrambling to get material to build product and managers struggling to keep orders on time.

The rubber manufacturer produces to customer orders and hence very few products are stocked. Jobs on the factory floor are run in a batch mode, usually comprising the entire order A work order and traveler are printed and sent along to each operation with the job. The first operation that the work order and traveler will go to is to the welding operation. When steel is ordered for a job, the material will sit on the floor for an average of three days before processing begins. The average set up time in welding for a modular panel is 30 minutes.  This must be done before each job is started. After the set up is complete, each frame only takes 4 minutes to weld.  The welding operation utilizes one operator on each of two shifts.

After welding, the frames are then sent to media blasting.  Large quantities of frames are in queue waiting for blasting. In this case, there are 10 pieces waiting for an average of 5 days. Blasting has a set up time of 30 seconds and each part takes on average 5 minutes to blast. The blaster is mainly operated only on second shift.

After the frames are blasted, they are placed on pallet and stored in any available floor space near the priming area. Again, large amounts of WIP wait in queue before being primed. The parts are left in queue until the rubber press is ready to utilize the frames. When a frame is needed in order to mould a part in the rubber press, the operator of the rubber press goes to the priming area and applies the adhesive only to the frames that are going to be pressed next. This is a pull system where frames are pulled from priming to the rubber press only when needed and only in the exact quantities needed. Because an operator primes only the frames that are needed and only when needed, there are no parts in queue between priming and rubber. The applying of the adhesive in the priming station takes 15 minutes with a set up time of one minute. When the rubber press operator finishes priming the frame it is taken by hand to the rubber press department. Here the frame is placed in a mould along with a specified amount of

raw natural rubber. The mold is placed in the press and the platens are closed. The cycle time for the rubber press is 45 minutes. This long cycle time gives the operator time to set up the next mould and prime the next frame. The changeover time for a new mould is one hour. To avoid making unnecessary changeovers, entire batches are processed through the press before changing to another mould. All the panels produced at the press are set on pallets until the entire order is completed.

[P.T.O]

After the entire order of panels is pressed, they are sent via forklift to the finishing department. The first operation in the finishing department is to remove flash at the trimming table. There are typically long wait times and large amounts of WIP in front of trimming because trimming is primarily performed only on first shift. The operator at the trim table will have the screens produced from both the second and third shift of the day before to trim before the panels produced that day can be trimmed. Trimming on average takes 7 minutes with a one minute changeover time. The next operation in the finishing department is clickering. Parts are moved from the trim table to the clicker and put into queue. Clickering primarily is performed only on the first and second shifts and therefore, large amounts of WIP are piled around the clicker as panels from multiple shifts are processed. In this case, 40 panels are queued in front of the clicker with an approximate wait in queue of a one-half day.

The last process in finishing is to saw the panels to the correct length. Rubber is hard to control dimensionally while processing. Therefore parts are made over-sized and then cut to length. The saw is manned only in one shift(second shift). Panels from the first and third shifts are piled around the saw. As recorded on the map, there are 20 panels in queue with a wait time of a one-half day. The time to saw a panel takes on average 6 minutes. The sawed panels are then placed on pallets on the floor around the saw. The panels will wait there until the order is ready to be prepared to ship which on average is three-quarters of a day.

When an order is ready to be shipped, a person from shipping will pick up the pallet from the finishing department and take it to the shipping department where the screen panels will be packed and made available for shipment. Shipments are made daily to the customers as needed.

Question:

Analyse the case given and as a lean management consultant offer your suggestions for improvement in the performance of the organization. Use the appropriate lean tools and techniques in your analysis.

2. 	Discuss in detail the primary elements of Lean Manufacturing.

(OR)

3. 	Discuss the need and importance of integrating lean strategy with culture and focus of the organization.

4. 	Explain the various sources of waste in an organization in detail and its impact on the organization performance.

(OR)

5. 	Discuss the steps involved in implementation of lean manufacturing in a typical organization.

6. 	What are the techniques available to assess the product flow in implementing lean manufacturing?

(OR)

7. 	Explain the importance of product flow and information flow in lean manufacturing.

8. 	“People play an important role in the success or failure of any improvement in the organization”. Discuss the importance of the statement in implementing lean manufacturing.

(OR)

9. 	What are the changes to be institutionalized for sustaining the efforts of implementing lean manufacturing?