Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination – June 2011

Subject Title:	AIRCRAFT STRUCTURES								       Time: 3 hours

Subject Code:	09AE202					       				       	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	In case of unsymmetrical bending, the direction of neutral axis is perpendicular to the plane of bending. (True / False)

2.	In an I- section, symmetrical about XX and YY axes, the shear centre lies at ______

3.	Major structural member of an aircraft wing are ______and ______.

4.	Longitudinal member of the fuselage structure which resist bending and axial loads is_______.

5.	The two analytical methods used to calculate the stress in the members are ______ and ______.

6.	Define stiffness factor.

7.	The strain energy stored by the body within the elastic limit, when loaded externally is called ______.

8.	As per Castigliano’s theorem, the deflection and rotation at a point is derived as ______ and ______ respectively.

9.	Slenderness ratio is defined as ______.

10.	As per maximum shear stress theory, stresses can be related with the equation ______.

PART – B (5 x 3 = 15 MARKS)

11.	Define the term Shear flow.  Give an expression for shear flow in a particular web section of the cross-section of a structure.

12.	Differentiate Monocoque and semi-Monocoque structure of fuselage.

13.	Differentiate statically determinate and indeterminate structures with examples.

14.	State and prove Maxwell Reciprocal Theorem.

15.	How are columns classified with different end conditions?

PART – C (5 x 15 = 75 MARKS)

16.	Find the shear centre of the channel section with projections shown in the figure.

b2

b1

t

t

t

h

(OR)

[P.T.O]

17.	A beam of T section (flange: 100mm x 20mm; Web: 150mm x 10 mm) is 2.5m in length and is simply supported at the ends. It carries a load 3.2 kN (at mid span) inclined at 20o to the vertical and passing through the centroid of the section. If E= 200GN/m2, calculate a) Maximum tensile stress  b) Maximum compressive stress  c) Deflection due to load and d) position of the neutral axis

18.	Explain in detail the major structural members of aircraft fuselage and also mention its significance.

(OR)

19.	Explain with neat sketch, the major load carrying members of a wing construction.

F

B

D

E

C

A

200N

400N

15m

15m

10m

600N

1200N

5m

15m

20.	Determine the axial forces in all the members of the pin-jointed truss loaded as shown in the figure:

(OR)

21.	Determine the support moments in the continuous beam shown in the fig by using three moment equations. Consider moment of inertia for AB and BC as I and for CD as 2I.

2I

I

3m

4m

5m

2m

2m

A

B

C

D

I

40kN

40kN

20kN/m

A

B

C

l/2

l/2

w / unit run

22.	A cantilever of rectangular section breadth b, depth d and of length l carries UDL spread from free end to the mid section of the cantilever as shown in fig. Using Castigliano’s theorem, find (a) the deflection due to shear at free end and (b) the deflection due to bending at free end.

(OR)

[P.T.O]

23.	Determine the deflection at the free end of the overhanging beam supported at A and B as shown in figure. Assume uniform flexural rigidity and UDL of 10 kN/m.

D

A

E

B

C

1.5m

1.5m

1m

0.5m

50kN

24.	Find the Euler crushing load for a hollow cylindrical cast iron column 20 cm external diameter, 25mm thickness, 6m long and is hinged at both the ends. Take E= 1.2 x 106 N/mm2, compare the load with the crushing load as given by Rankine’s formula, taking maximum possible compressive stress (σc) as 550 N/mm2 and a= 1/1600. For what length of the column would these two formulae give the same crushing load?

(OR)

25.	According to the theory of maximum shear stress, determine the diameter of a bolt which is subjected to an axial pull of 9KN together with a transverse shear force of 4.4 KN. Elastic limit in tension is 225 N/mm2, factor of safety = 3 and Poisson’s ratio = 0.3.