Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	MECHANICS OF DEFORMABLE BODIES II 			       Time: 3 hours

Subject Code:	09CE206	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Write down the relationship between actual beam and conjugate beam.

2.	What is the condition for maximum deflection in a beam?

3.	Define bulking load.

4.	Write the secant formula for eccentrically loaded column.

5.	What is a thick cylinder?

6.	What is elastic failure?

7.	What are curved beams?

8.	What is shear flow?

9.	Define product of inertia.

10.	What is neutral axis?

PART – B (5 x 3 = 15 MARKS)

l/2

l/2

W

w/m

I

2I

11.	Draw the (M/EI) diagram for the beam shown

12.	A column one end fixed other end hinged is made up of hollow section 200 mm outer dia and 150mm inner dia.  The length of the column is 2.0 m.  Calculate the slenderness ratio of the column.

13.	What is shrink fit?  What are its advantages?

14.	State and explain the maximum shearing stress theory.

15. 	When shall unsymmetrical bending come into picture?

PART – C (5 x 15 = 75 MARKS)

2 m

3 m

2 m

10 kN

15 kN

16.	Calculate the value of maximum deflection in the beam shown below.  EI is constant.  Use Macaulay’s method.

(OR)

17.	Calculate the deflection at the mid-span of the beam shown.  Use any method.

w/m

I

2I

l/2

l/2

[P.T.O]

18.	Determine the cross-sectional dimensions of the hollow circular column for the following conditions.

Safe load on the column	=	250 kN

Factor of safety			=	3

fc						=	550 N/mm2

						=	(1/1600)

Both ends of the column is fixed

Inner diameter			= 	0.75 outer diameter

(OR)

19. 	Following are the particulars pertaining to an eccentrically loaded column.

Length of the column		=	4.0 m

Both ends of the column is fixed

Load on the column		=	150 kN

Eccentricity				=	25 mm from the axis

Hollow circular section 200 mm outer dia. and 20 mm thick.

E						=	94 kN/mm2

Find the extreme stresses on the column section.  If no tension is to occur in the column section, what is the maximum eccentricity?

20. 	A compound tube is composed of a tube of 200 mm internal diameter and 20 mm thick shrunk on a tube of 200 mm external dia. and 20 mm thick.  The radial pressure at the junction is 6N/mm2.  The compound tube is subjected to an internal pressure of 80 N/mm2.  Find the variation of the hoop stress over the wall of the compound tube.

(OR)

21. 	Explain the phenomenon of failure of a material when subjected to a complex state of stress.

22. 	Locate the shear centre for the channel section shown.

40 mm

1 mm

100 mm

1 mm

1 mm

40 mm

(OR)

23. 	A crank hook has the dimensions as show in figure.

A

B

70 mm

P

50 mm

70 mm

100 mm

Section at AB

Determine:  (a) the maximum load P that will not exceed stress of 120 N/mm2 at A.  What is the corresponding stress at B?

[P.T.O]

24.	Calculate the Principal moment of inertia for the section shown.

8 mm

8 mm

80 mm

80 mm

(OR)

25. 	A cantilever beam 40 mm x 40 mm section, 2.0 m length is subjected to a load of 1 kN acting at the free end as shown.  Determine the maximum stress is the beam.

P

60o