Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	GEOTECHNICAL ENGINEERING	 					       Time: 3 hours

Subject Code:	09CE212	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is residual soil?

2.	Define void ratio.

3.	Define plasticity index.

4.	Write down the size of sand.

5.	Define Darcy’s law.

6.	Define seepage.

7.	What is consolidation?

8.	Define co-efficient of compressibility.

9.	Draw Mohr’s failure envelope of sandy soil.

10.	Write down the factor of safety equation with respect to friction.

PART – B (5 x 3 = 15 MARKS)

11.	Differentiate void ratio and porosity.

12.	What are the different soil classification systems?

13.	Explain total, neutral and effective stresses.

14.	What are the components of settlement?

15.	Explain shear strength parameters.

PART – C (5 x 15 = 75 MARKS)

16.	One cubic meter of wet soil weighs 19.80kN.If the specific gravity of soil particles is 2.70 and water content is 11%, find the void ratio, dry density and degree of saturation.

(OR)

17. 	Explain the tests for specific gravity and water content.

18.	Explain atterberg limits.

(OR)

19.	Explain the classification of soil by IS classification of soil and distinguish if a material is GW or GP or GM or GC.

20.	A saturated sand layer over a clay stratum is 5 m depth. The water is 1.5m below ground level. If the bulk density of saturated sand is 17.66 kN/m3, calculate the effective and neutral pressure on the top of the clay layer.

(OR)

21.	Define flow net.  Explain the characteristics and suitable procedure for drawing flow net.

[P.T.O]

22.	A saturated soil has a compression index of 0.25. Its void ratio at a stress of 10kN/m2 is 2.02 and its permeability is 3.4x10-7 mm/s. Compute:

a.	Change in void ratio if the stress is increased to 19kN/m2.

b.	Settlement in if the soil stratum is 5m thick and

c.	Time required for 40% consolidation if drainage is one-way.

(OR)

23.	A layer of soft clay is 6m thick and lies under a newly constructed building. The weight of sand overlying the clayey produces a pressure of 260kN/m2 and the new construction increases the pressure by 100 kN/m2. If the compression index is 0.5, compute the settlement. Water content is 40% and specific gravity of grains is 2.65.

24.	The following data relate to a triaxial compression tests performed on a soil sample.

| Test No. | Chamber pressure | Max.deviator stress | Pore pressure at maximum deviator stress |
| --- | --- | --- | --- |
| 1 | 80 | 175 | 45 |
| 2 | 150 | 240 | 50 |
| 3 | 210 | 300 | 60 |

Determine the total and effective stress parameters of the soil.

(OR)

25.	Explain the friction circle method of stability analysis.