Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	APPLIED HYDRAULICS AND FLUID MACHINES   	       Time: 3 hours

Subject Code:	09CE216	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What do you mean by shooting flow?

2.	Define Uniform flow in open channel.

3.	What is laminar sublayer?

4.	Define Reynolds number.

5.	Write impulse momentum equation.

6.	What is fluid jet?

7.	Define specific speed of turbine.

8.	Give an example for reaction turbine.

9.	What do you mean by priming of pump?

10.	Define the slip in pump.

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between mild slope and steep slope in open channel.

12.	Differentiate between Stream lined and Bluff bodies.

13.	Derive an expression for the fluid jet striking a stationery normal flat plate.

14.	What are the various types of draft tube in turbines?

15.	Explain the minimum starting speed of pump.

PART – C (5 x 15 = 75 MARKS)

16.	Design an earthen trapezoidal channel for water having a velocity of 0.6 m/sec, side slope is 1:1.5 and quantity of water is 3 m3/sec. Assume Chezy’s constant as 65.

(OR)

17.	Derive an expression for the variation of depth along the length of bed of the channel for a gradually varied flow in open channel.

18.	The velocity distribution in the boundary layer is given by u / U = 2 (y/δ) - (y/δ)2. Find displacement thickness, momentum thickness and energy thickness.

(OR)

19.	A jet plane having a wing area of 20m2 and weighing 25 kN flies at 950 km/hr speed. The engine develops 8500 kW and has a mechanical efficiency of 60%. Determine the lift and drag coefficients for the wind. Take the specific weight of air is 12 N/m3.

20.	A jet of water of 60 mm diameter strikes a curved vane at its centre with a velocity of 18 m/sec. The curved vane is moving with a velocity of 6 m/sec in the direction of the jet. The jet is deflected through an angle of 1650. Assuming the plate to be smooth, Find i) thrust on the plate in the direction of jet, ii) power of the jet and iii) efficiency of the jet.

(OR)

21.	Derive an expression for the force exerted by the jet on the moving plate held at normal to the jet and inclined to the direction of jet.

[P.T.O]

22.	The propeller reaction turbine of runner diameter 4.5m is running at 48 r.p.m.  The guide blade angle at inlet is 1450 and the runner blade angle at outlet is 250 to the direction of vane. The axial flow area of water through the runner is 30 m2. If the runner blade angle at inlet is radial, determine a) Hydraulic efficiency of the turbine b) Discharge through the turbine and c) power developed by the runner.

(OR)

23.	The following data relate to a Pelton wheel turbine is given below:

Head at the base of the nozzle = 82 m, diameter of the jet = 100 mm, discharge of the	 nozzle = 0.3 m3/s, shaft power = 206 kW. Determine the power lost in nozzle and power lost due to hydraulic resistance in water.

24.	The impeller of centrifugal pump is of 30 cm diameter and 5 cm width at the periphery, and has blades whose tip angles incline backwards 600 from the radius. The pump delivers 17 m3/min and the impeller rotates at 1000 r.p.m. Assuming that the pump is designed to admit radially, Calculate i) speed and direction of water as it leaves the impeller ii) torque exerted by the impeller on water iii) shaft power required and iv) lift of the pump.

(OR)

25.	The plunger diameter and storke length of a single-acting reciprocating pump are 300 m and 500 mm. The speed of the pump is 50 r.p.m. The diameter and length of delivery pipe are 150 mm and 55 m. If the pump is equipped with an air vessel on the delivery side at the centre line of the pump, find the power saved in overcoming friction in the delivery pipe. Take friction co-efficient is  f = 0.01.