Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	STRUCTURAL DYNAMICS 						              Time: 3 hours

Subject Code:	09CE308		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	For a system having a mass of 10 kg and spring constant 12 kN/m, the amplitude decreases 	from an initial value of 10mm to 2 mm after six consecutive cycles.  Find the damping 	coefficient and absolute damping.												(10)

b.	A SDOF system with a mass of 50kg and stiffness 20 kN/mm and damping 150Ns/m is initially at rest.  If the initial velocity is 100 mm/s, find an expression for the subsequent displacement.  Hence find velocity at t = 2 sec.									(10)

2.	Determine the state response of a simply supported beam of span L, flexural rigidity EI, subjected to a concentrated force of Q suddenly applied at one third of its span.  Proceed from first principles.

(OR)

3.	An idealized SDOF system has stiffness of 7150 N/m and its mass is 20 kg.  Determine the response of the mass by numerical evaluation of Duhamel’s integral procedure, if an external forcing function, F(t) = 260 sinπt0.6is applied on the system.  The system is undamped and is initially at rest.  Take t = 0.1 sec.  Show minimum of 4 displacement values.

4.	Explain Rayleigh’s variational method with an example.

(OR)

5.	a.	Find the fundamental frequency of a uniform cantilever beam of length L and mass m 	using suitable shape function.													(10)

b.	Explain response spectrum.													(10)

6.	A single bay three storeyed building frame has stiffness 40 kN/mm for all columns.  The concentrated mass at all floor level is 1000 kg.  Find natural frequencies and mode shapes.

(OR)

7.	Write short notes on the following:

a.	Stodala iteration techniques	b.	Holzer method	c.	Dunkerlay’s equation	   (7+7+6)

8.	Explain the procedure to solve a multi storeyed building frame for

a.	Free vibration with damping		b.	Forced vibration without damping		   (10+10)

(OR)

9.	Write short notes on the following:

a.	Central Difference method		b.	Houbolt method		c.	Newmark method	   (7+7+6)