Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	PRESTRESSED CONCRETE STRUCTURES 		              Time: 3 hours

Subject Code:	09CE311		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Write a detailed note on post tensioning systems.				     					(12)

b.	A prestressed concrete beam 400mm wide and 600 mm deep has a span of 6 meters. The 	beam is prestressed with a tendon bent as shown in figure. The external load on the beam 	consists of a concentrated load of 180kN at mid span. If the effective prestressing force is 	1200 kN, calculate the extreme stresses for the mi span section.    					(8)

2.	a.	Figure shows the section of a prestressed concrete beam of span 9 meters. The beam has to 	carry a superimposed load of 20 kN per meter. The prestressing force is transmitted by 	tendons in two cables. In each cable the tendons consist of 14 wires of 5mm diameter 	subjected to an initial stress 1000N/mm². Analyse the beam section at the centre for the 	stresses induced before and after the application of the superimposed load. Allow 15% loss 	of prestress.								     									(12)

b.	Write the steps involved in design of a I section prestressed concrete beam.	      		(8)

(OR)

[P.T.O]

3.	A post tensioned concrete beam of rectangular section 250 mm wide and 500mm deep has a span of 12.50m and carries a superimposed load of 8.5 kN/m. The tendon is provided with a parabolic profile with a central dip of 180mm and with no eccentricity at the ends. The effective prestressing force in the tendon is 750 kN. Determine

a.	The principal stresses at the supports

b.	The principal stresses at the supports without prestress.

Take weight of concrete = 24 kN/m³

4.	a.	How is the end block analyzed by Guyon’s method?                                             		(8)

b.	A prestressed concrete beam 250mm wide and 650mm deep is subjected to an effective prestressing force of 1360kN along the longitudinal centroidal axis. The cables may be assumed to be symmetrically placed over mild steel anchor plate in an area 150mm X 350mm. Design the end block.

Take cube strength of concrete at transfer f ci  = 30 N/mm²

Characteristic strength of concrete f ck  = 30 N/mm²

Assume initial prestressing force = 1.2 times the effective prestressing force.              (12)

(OR)

5.	Figure shows the cross section of a composite beam which consists of a 300mmX900mm precast stem and cast- in- situ flange 900mm X 150mm. The stem is post tensioned unit with an initial prestressing force of 2500 kN. The effective prestress available after making deduction for losses is 2200 kN. The dead moment due to the weight of the flange is 125 kN. After the hardening of the flange concrete, the composite section has to carry a live load which produces a bending moment of 700 kNm.

Determine the stress distribution in concrete at the various stages of the loading.  \

6. 	a.	The cable profile for a two span continuous beam is shown in figure, the prestressing force 	being 1200 kN. Locate the pressure line due to the prestressing force alone.       		(12)

b.	Write the advantages of continuous beam.				   	       					(8)

(OR)

[P.T.O]

7.	A post tensioned continuous beam consists of two spans each 20 meters long. The external loading other than the dead load of the beam is 20 kN/m. Design the beam.

8.	a.	A prestressed concrete tank of diameter 20 meter has to resist an internal pressure head of 	5 meters of water. Find the reinforcement per meter height and the thickness of concrete 	required. Strength of mortar coating over the wires may be ignored.

Take

f ck   =  ultimate strength of concrete = 35 N/mm²

f c =  Safe stress in concrete = 0.5 f ck  at transfer. Stress in concrete at servicing condition shall remain compressive.

f c  =  1000 N/mm², Loss of prestress = 18%, m= 8                                              		(12)

b.	Draw the typical composite cross sections of prestressed concrete beam.              		(8)

(OR)

9.	Explain in detail the prestressed concrete piles and concrete poles.