Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	ADVANCED CONCRETE TECHNOLOGY 			              Time: 3 hours

Subject Code:	09CE316		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

A mix is required to be designed for M 20 grade of concrete, which is to be used in lightly reinforced slab. Other details are as given below:

Batching =40 N/mm2.

Workability (Low)= 35- 75mm slump

Exposure = mild

w/c ratio = 0.55

Cement content = 300 kg/m3

Size of aggregate =20mm

Type of aggregate = uncrushed

Type of sand = river sand

Cement = OPC43 grade

Test results

Compressive strength = 39.5 N/mm2

Sand grading = zone I

Aggregate grading = 20- 5mm graded aggregate

Specific gravity of sand and aggregate = 2.7

Water absorption of sand and aggregate = 0.8 % and 0.6 %

Bulk density of dry sand and dry aggregate = 1.53 and 1.48 kg/ litre.

2.	a.	List the different types of cement. Explain the procedure for standard consistency test and 	setting time test.															   (4+4+2)

b.	State the properties and uses of

i)	Portland blast furnace slag cement		ii)	Blended cement					(5+5)

(OR)

3.	a.	How does grading of aggregates influence the properties of concrete? Explain the 	procedure to find elongation index.												(12)

b.	Discuss about the alkali aggregate reaction.										(8)

4.	a.	What are the factors affecting workability?										(4)

b.	What is meant by gunite? Explain the process used in gunite.						(8)

c.	Explain about preplaced concrete.												(8)

(OR)

5.	a.	Why is compaction of concrete important? What are the various methods of compaction? 	Write any one in detail.														(12)

b.	What are all the different types of curing of concrete?								(4)

c.	What are the factors affecting the shrinkage of concrete?							(4)

[P.T.O]

6.	a.	How does permeability affect durability of concrete?								(5)

b.	Write a short note on concrete in sea water.										(5)

c.	How will you find the compressive strength of existing concrete in a building using 	schemidt hammer? Explain.													(10)

(OR)

7.	a.	Describe a test to determine tensile strength of concrete. What are all the factors which 	affect permeability?															(10)

b.	How is modulus of elasticity determined?										(10)

8.	Explain the step by step procedure for

a.	BIS method		b.	UK and Australia code method

(OR)

9.	Write short notes on:																(5x4)

a.	High strength concrete		b.	High performance concrete

c.	Self compacting concrete		d.	Ready mix concrete	e.	Pumped concrete