Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title : 	BASIC ELECTRICAL ENGINEERING					      Time : 3 hours

Subject Code:	09EE101	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	If the diameter of a conductor is doubled, its resistance becomes __________ times.

2. 	1 kWh = __________J.

3. 	MMF produced by current I amps flowing through a coil having N turns = __________.

4. 	Self inductance of coil of 1000 turns carrying 1 amp producing flux 25 mWb = __________.

5. 	The power factor of a purely capacitive circuit is __________.

6. 	Peak factor of a sinusoidal voltage = __________.

7. 	The transformer works on the principle of __________.

8. 	The rotor frequency of a 4 pole 50Hz induction motor running at 1470 rpm = __________.

9. 	In the case of an indicating instrument, the needle is steady when deflecting

torque = __________.

10. 	Earthing is required for protection against __________.

PART – B  (5 x 3 = 15 MARKS)

11.  	a.	State Ohm’s Law. 															(2)

b.	What is the resistance of a 250 V, 1000 W heater coil?							(1)

12. 	What are the Faraday’s Laws of Electromagnetic Induction?

13.	Give five comparisons between overhead and underground distribution systems.

14.	Explain the principle of operation of transformer.

15.	What is damping torque?  How is it produced in an indicating instrument?

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	State Kirchhoff’s Laws and illustrate them using a circuit.              					(8)

b. 	Three resistors of 12 ohm, 10 ohm and 6 ohm are connected in parallel. If 5 A is flowing through the 12 ohm resistor, what resistance is to be connected in parallel with this combination so that the total current drawn by this circuit is 25 A? How much energy is consumed by this circuit in one hour?  											(7)

(OR)

17.	a.	Obtain expressions for the equivalent star network resistances for a given delta network.

(10)

b.	Obtain the equation for the voltage across any resistance in a series circuit having ‘n’ number of different resistances.												(5)

18.	a.	Define the following:								                             			(6)

i.	Magnetic Flux Density.	ii.	Reluctance.		iii.	Permeance.

b.	Derive the expression for the flux produced in a toroidal iron ring of mean radius R meters, area of cross section a sq. m, having a coil of N turns carrying a current of I amps.	(9)

(OR)

[P.T.O]

19.	a. 	State Fleming’s left hand rule.     						                              		(3)

b. 	Derive the expression for self inductance of a coil in terms its number of turns and reluctance of the magnetic circuit.    											(8)

c. 	A straight conductor 25 cm long carries 100A and lies perpendicular to a uniform field of 0.5 Wb/sq.m. Find the mechanical force acting on the conductor.					(4)

20.	With a neat sketch, describe the operation of a nuclear power plant.

(OR)

21.	a. 	Derive the RMS value, average value, form factor and peak factor of a sinusoidal

alternating current quantity.										          		(10)

b. 	Find out the equation for the instantaneous value of voltage of 240V 50Hz supply.	(5)

22.	Explain the principle, construction and working of a three phase induction motor.

(OR)

23.	Explain the principle and construction of a DC generator with a neat diagram.

24.	With a neat diagram, explain the construction and working principle of induction type energy meter.

(OR)

25. 	a. 	A lamp has to be controlled from two locations. Propose a scheme and explain the

operation.      																(7)

b. 	Explain with neat diagram fluorescent lamp wiring.				            			(8)