Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	SIGNALS AND SYSTEMS 								       Time: 3 hours

Subject Code:	09IT207		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define a signal.

2.	The signals that are defined at all instant of time are known as __________.

3.	An LTI System is one which posses two basic properties __________ and __________.

4.	Define a Causal System.

5.	If the periodic signal x(t) is having even symmetry then the Fourier expansion contains __________ terms only.

6.	The Fourier expansion of a half wave symmetric periodic signal contains __________ harmonics.

7.	What is the transfer function of a Zero-order hold?

8.	Define Nyquist rate.

9.	The inverse z-transform of x(z) is defined as__________.

10.	State time-shifting property of z-transform.

PART – B (5 x 3 = 15 MARKS)

11.	What are the basic operations on signals?

12.	What are the properties of convolution?

13.	Define Fourier transform and inverse Fourier transform.

14.	State the initial value theorem and final value theorem of Z-transform.

15.	State sampling theorem.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Define and sketch a unit step and delta function.									(6)

b.	Check whether the following systems are causal or not.							(6)

i)	y(n) = x(n)+[1/x(n-1)]		ii)	y(t) = x(t-2)+x(2-t)

c.	Check whether the following system is linear or not.	y(n) = nx(n)					(3)

(OR)

17.	a.	Check whether the following system is linear or not.

i)	y(n) = Ax(n)+B	ii)	y(n) = cos[x(n)]	iii)	y(n) = x(n)x(n)					(3x3)

b.	Check whether the following systems are time variant or time invariant.

i)	y(t) = odd(x(t))		ii)	y(n) = x(n)x(n-1)									(3+3)

18.	Explain the classifications of continuous time system in detail.

(OR)

19.	a.	Define the response of LTI continuous time system using convolution.				(7)

b.	Derive the zero-input response of LTI Discrete time System in time domain.			(8)

20.	Derive the transfer function of LTI continuous time system in frequency domain.

(OR)

[P.T.O]

21.	Determine the Fourier series representations of the following discrete time signals.

a.	x(n) = 2cos√3n		b.	x(n) = 4 cos (n/2)	c.	x(n) = 3e j5n/2				(3x5)

22.	Explain the effect of aliasing in frequency spectrum due to sampling.

(OR)

23.	Using Laplace transform, determine the forced response of the system described by the equation:  5 dy(t)/dt + 10y(t) =2x(t) : for the input x(t) =2u(t)

24.	Determine the Z-transform and their ROC of the following discrete time signals.

a.	x(n) = u(n)	b.	x(n) = 0.5nu(n)		c.	x(n) = 0.5nu(n)+0.8u(-n-1)				(3x5)

(OR)

25.	Explain the properties of Z-transform.