Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	ENGINEERING MECHANICS 							      Time: 3 hours

Subject Code:	09ME201	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Which law of Newton is interpreted in terms of equilibrium concept?

2.	Define a particle.

3.	The resultant force of a couple is ____________.

4.	The force of a person’s hands on a T shaped handle is an example of ____________.

5.	The centroid of a plane curve is physically located on the curve. (True/False)

6.	Give the expression for mass moment of inertia of a sphere of radius ‘r’ and mass ‘m’.

7.	What is meant by a uniform rectilinear motion?

8.	In a perfectly elastic impact, ____________ and____________ of the two particles are conserved.

9.	Coefficient of kinetic friction is greater than coefficient of static friction. (True/False)

10.	Give an example for applications where friction forces are detrimental.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the principle of moments.

12.	State the conditions of equilibrium of a rigid body in two dimensions.

13.	Define product moment of inertia and discuss about its sign.

14.	State D’Alembert’s principle.

15.	What is meant by a self locking screw?

PART – C  (5 x 15 = 75 MARKS)

C

D

y

x

z

B

A

0.76 m

0.91 m

9.07 kg

0.91 m

1.37 m

0.46 m

0.46 m

16.	Determine the tension in the cables AB, AC and AD if the crate shown in Figure is weighing 9.07 kg.

(OR)

[P.T.O]

800N

y

x

z

A

B (5, 0, 2)

C

(-2,0,-3)

1000N

P

(-4, 0, 0)

(0, 0, 0)

O

17.	A tripod is acted upon by forces at P as shown in Figure. Determine the forces in the legs of the tripod if the legs rest on the ground at A, B and C whose coordinates with respect to O are as shown. The height of the point P is 10 m above O.

18.	 Determine the resultant of the four forces tangent to the circle of radius 3 m shown in figure. What will be its location with respect to the center of the circle? Also calculate the x and y intercepts.

80N

50N

150N

100N

45

(OR)

100N

600N

A

B

C

D

3m

2m

2m

200N

45

19.	Determine the reactions at the supports for the beam loaded as shown in figure.

20.	Determine the centroidal coordinates of the area shown in figure.

80mm

100mm

100mm

80mm

60mm

40mm

40mm

20mm

X

Y

O

(OR)

[P.T.O]

21.	Calculate the mass moment of inertia of the lamina of weight 0.5 N and uniform thickness 1 cm shown in figure with respect to the centroidal axis.

22.	A bullet is fired with an initial velocity of 200 m/s at an angle of inclination of 60 from the horizontal from the top of a hill which is 300 m above the ground level.

a.	Find the maximum elevation reached by the bullet above the ground level

b.	Horizontal distance at which the bullet will strike the ground

c.	The magnitude and direction of velocity with which the bullet strikes the ground.

(OR)

23.	a.	A body of mass 0.5 kg is moving on a smooth horizontal plane along the arc of a circle of

10 m radius with a constant tangential acceleration of 5 m/s2. At a given instant the body has a speed of 4 	m/s. What force P applied to the body parallel to the plane is required to produce the instantaneous 	motion indicated?										(8)

b.	A flywheel starting from rest attains a speed of 600 rpm in 30 seconds with uniform acceleration. Determine the number of revolutions made by flywheel at the end of 20 seconds.																		(7)

24.	A uniform ladder of 7 m length rests against a vertical wall with which it makes an angle of 45. The coefficient of friction between the ladder and the wall is 1/3 and that between the ladder and floor is ½. If a man whose weight is ½ of that of the ladder, ascents it, how high will he be when the ladder slips?

(OR)

25.	A block weighing 200 N rests on a plane inclined at 45 to the horizontal. The block is tied by a horizontal string as shown in Figure. The block is in equilibrium when the tension in the string is 70 N. Determine the friction force, the normal reaction of the plane and the coefficient of friction between the block and the plane.

String

45