Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	THERMAL ENGINEERING 							       Time: 3 hours

Subject Code:	09ME203	       						               	       Maximum Marks: 100

(Use of Standard Thermodynamic Tables, Mollier Diagram, Psychometric Chart and Refrigerant Property Tables are permitted)

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the range of compression ratio for diesel engine?

2.	Name any two pollutants which are the cause for pollution.

3.	State the function of fixed blades in turbines.

4.	Where is by-pass governing more suitable?

5.	Which type of compression is the best in reciprocating compressor?

6.	Give the applications of compressor.

7.	In a vapour compression refrigeration system, where will the highest temperature occur?

8.	Write any two commonly used refrigerants.

9.	What is meant by Humidification?

10.	What are the different types of Air conditioning system available in the market?

PART – B (5 x 3 = 15 MARKS)

11.	What is meant by detonation?

12.	What are the different methods of compounding?

13.	Give the expression for inter cooler pressure.

14.	What is sub cooling with respect to refrigeration cycles?

15.	What is meant by sensible cooling and sensible heating?

PART – C (5 x 15 = 75 MARKS)

16. 	a.	Explain the principle of operation of four stroke cycle spark ignition engine with a neat 	sketch.                                                                                                              			(10)

b.	Differentiate two stroke and four stroke engines.                                                   		(5)

(OR)

17.	a.	Explain with a neat sketch, the cooling systems used in I.C engines               		(10)

b.	Explicate the phenomenon of knocking in CI engine                                              	(5)

18.	a.	The blade speed of a single ring impulse blading is 300 m/sec and nozzle angle is 20º.  The 	heat drop is 625 kJ/kg and nozzle efficiency is 0.85. The blade discharge angle 30º and the 	machine develop 50 kW, when consuming 300 kg of steam per hour. Calculate axial thrust 	on the blade.                                                                          						(10)

b.	How is pressure – velocity compounding done?                                                    		(5)

(OR)

19.	a.	With neat sketches explain the principles of operation of impulse and reaction turbines.

(10)

b.	Write the speed ratio and stage efficiency as applied to steam turbines.                		(5)

[P.T.O]

20.	a.	Single stage reciprocating compressor receives air at 25m3/min at 1bar, 15ºC and 	discharges it at 15bar. Assume the value of n for compression as 1.35 and volumetric 	efficiency as 0.75. Determine (i) Theoretical power required (ii) Maximum air temperature.

(10)

b.	Describe the working principle of an axial flow compressor                            		(5)

(OR)

21.	a.	A two stage compressor works between 1 bar and 16 bar. The inlet temperature is 30ºC 	Inter cooling is perfect and compression is isentropic. Find the intermediate pressure and 	also find the work done per kg of air with and without inter cooling                  		(10)

b.	Discuss the merits and demerits of rotary compressor and reciprocating compressor.	(5)

22.	a.	Describe the operation of a vapour compression refrigeration system with a neat sketch.

(10)

b.	Name any five properties of good Refrigerant.                                                 		(5)

(OR)

23.	a.	A refrigeration system operates with condensing and evaporating temperatures of 30ºC and 	-5 ºC. There is no sub cooling and refrigerant is dry saturated at the compression.  Find 	theoretical COP.																(10)

| Temperature | Enthalpy (kJ/kg) |  | Entropy (kJ/kg K) |  |
| --- | --- | --- | --- | --- |
|  | Liquid | Vapour | Liquid | Vapour |
| -5ºC | 158.3 | 1431.9 | 0.63 | 5.41 |
| 30ºC | 323 | 1465.4 | 1.202 | 4.984 |

b.	Give the applications of refrigeration systems.                                                   		(5)

24.	a.	Atmospheric air at 760 mm of Hg has 45ºC dry bulb temperature and 30ºC wet bulb 	temperature using psychrometric chart, calculate the following						(10)

i)	Relative humidity		ii)	Humidity Ratio		iii)	Dew point temperature

iv)	Enthalpy			v)	Specific volume of air.

b.	What is the difference between air conditioning and refrigeration?                			(5)

(OR)

25.	a.	Describe with a sketch about the window air conditioning system.                  		(10)

b.	What is meant by Evaporative cooling?                                                           			(5)