Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	MECHANICS OF MACHINES - I 						       Time: 3 hours

Subject Code:	09ME205	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Write down the Kutchback criterion for a mechanism.

2.	Define degrees of freedom.

3.	What is the condition for maximum power transmission of a belt?

4.	What are the assumptions made in the derivation of torque of a clutch?

5.	What are the types of cam?

6.	Define pressure angle in cam.

7.	Define module.

8.	What is meant by back lash?

9.	Define sensitiveness of a governor.

10.	Write down the expression for gyroscopic couple.

PART – B (5 x 3 = 15 MARKS)

11.	What is meant by double slider crank chain? Write down the inversions of double slider crank chain.

12.	What are the factors to be considered in the selection of a belt drive?

13.	List different types of motion of a follower.

14.	What is meant by reverted gear trains? Give the applications of it.

15.	Differentiate flywheel and governor.

PART – C (5 x 15 = 75 MARKS)

16.	Explain with a neat sketch any three inversions of a single slider crank chain.

(OR)

17.	The crank of a slider crank mechanism rotates clockwise at a constant speed of 300 rpm. The crank is 150 mm and the connecting rod is 600 mm long. Determine (a) linear velocity and acceleration of the midpoint of the connecting rod (b) angular velocity and angular acceleration of the connecting rod at crank angle of 45 from inner dead centre position.

18.	An open belt running over two pulleys 240 mm and 600 mm diameter connects two parallel shafts 3 m apart and transmits 4 kW from the smaller pulley that rotates at 300 rpm. The coefficient of friction between the belt and the pulley is 0.3 and the safe the safe working tension is 10 N/mm widths. Determine (a) minimum width of the belt (b) initial belt tension (c) length of the belt required.

(OR)

19.	A leather faced conical clutch has a cone angle of 30.  If the intensity of pressure between the contact surfaces is 0.35 N/mm2 and the breadth of the conical surface is not to exceed 1/3rd of the mean radius, find the dimensions of the contact surfaces to transmit 22.5 kW at 2000 rpm. Assume uniform rate of wear and take coefficient of friction as 0.15.

[P.T.O]

20.	A cam with a minimum radius of 25 mm, rotating clockwise at a uniform speed is to be designed to give a roller follower, at the end of valve rod, a simple harmonic motion described below:

a.	to rise the valve through 50 mm during 120  rotation of the cam.

b.	to keep the valve fully raised through next 30 

c.	to lower the valve during next 60

d.	to keep the valve closed during the rest of the revolution i.e. 150 

The diameter of the roller is 20 mm and the diameter of the cam shaft is 25 mm.  Draw the profile of the cam when the line of stroke of the valve rod passes through the axis of the cam shaft.

(OR)

21.	Draw the profile of the cam when the roller follower moves in a cycloidal motion during outstroke and return stroke as given below:

a.	Outstroke with a maximum displacement of 31.4 mm during 180 of the cam rotation

b.	Return stroke for the next 150  of the cam rotation.

c.	Dwell for the remaining 30 of cam rotation.

The minimum radius of the cam 15 mm and the roller diameter of the follower is 10 mm. The axis of the roller follower is offset by 10 mm towards the right from the axis of the cam shaft.

22.	A pinion having 30 teeth drives a gear having 80 teeth. The profile of the gears is involutes with 20  pressure angle, 12 mm module and 10 mm addendum. Find the length of path of contact, arc of contact and contact ratio.

(OR)

23.	In an epicyclic gear train, an arm carries two gears A and B having 36 and 45 teeth respectively. If the arm rotates at 150 rpm in the anticlockwise direction about the centre of the gear A which is fixed, determine the speed of the gear B. If the gear A instead of being fixed makes 300 rpm in clockwise direction, what will be the speed of the gear B?

24.	A Proell governor has equal arms of length 300 mm. The upper and lower ends of the arms are pivoted on the axis of the governor. The extension arms of the lower links are each 80 mm long and parallel to the axis when the radii of rotation of the balls are 150 mm and 200 mm. the mass of each ball is 10 kg and the mass of the central load is 100 kg. Determine the range of speed of governor.

(OR)

25.	Explain with neat sketch the effect of gyroscopic couple on an aero plane.