Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	ADVANCED THERMODYNAMICS  				              Time: 3 hours

Subject Code:	09ME302		         						              Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Briefly discuss about some applications of entropy principle.						(10)

b.	A room is maintained at 27 0C while the surroundings are at 20C. The temperature of the 	inner and outer surfaces of the wall (k=0.71 W/mk) are measured to be 210C and 60C, 	respectively. Heat flows steadily through the wall 5m X 7m cross-sections and 0.32 m in 	thickness. Determine (a) the rate of heat transfer through the wall (b) the rate of entropy 	generation in the wall and (c) the rate of total entropy generation with this heat transfer 	process.																		(10)

2.	a.	Derive Maxwell’s equations.													(10)

b.	Derive the equation dp/dT=1/T(v(f) -v(t) )										(10)

(OR)

3.	a.	Give the Gibbs phase rule for a nonreactive system. Why is the tripple point of a substance 	no variant?																	(12)

b.	State the conditions of stability.												(8)

4.	a.	Define fugacity and activity.													(8)

b.	Briefly discuds about compressibility chart.										(12)

(OR)

5.	a.	What are the various types of equilibrium?										(10)

b.	Derive the conditions for local equilibrium.										(10)

6.	a.	Explain the estimation of stochiometric air.										(8)

b.	A certain cooking gas is found to be a mixture of 20% propane and 80% butane on volume basis. Find the stochiometric air. The above gas is burnt steadily and completely in a burner. Give the ratio of gas to dry air is 1:40 by volume, find the excess air and % theoretical air.																(12)

(OR)

7.	a.	How is the first law of thermodynamics for a system applied to combustion.			(10)

b.	Write short notes on adiabatic combustion temperature.							(10)

8.	 Explain about the perfect gas model, based on  specific heats.

(OR)

9.	A mass of 0.25 kg of an ideal gas has a process of 300 kpa, a temperature of 800C, and a volume of 0.07 m3.  The gas undergoes an irreversible adiabatic process to a final pressure of 300 kpa and final volume of 0.10 m3, during which the work done on the gas is 25kJ. Evaluate the Cp and Cv of the gas and the increase in entropy of the gas.