Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	DIGITAL SIGNAL PROCESSING 						       Time: 3 hours

Subject Code:	10MT203	       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Find the fundamental period number for g(n)=e−j 2 π n3

2.	State the conditions for the causality and stability of a DT System.

3.	Draw the basic butterfly structure in DFT-FFT algorithm and represent the input-output relation.

4.	Calculate the multiplication reduction factor, α in computing 1024 point DFT, in a radix-2 FFT algorithm.

5.	Write the magnitude function of Butterworth filter. What is the effect of varying order of N on magnitude and phase response.

6.	Find the order and poles of butterworth LPF that has 3dB bandwidth of 500 Hz and an attenuation of 40 dB at 1000 Hz.

7.	What do you understand by A/D conversion noise?

8.	List the important characteristics of physically realizable filters.

9.	Compare fixed point representation of coefficients with floating point representation.

10.	Give the expression for signal to quantization noise ratio and calculate the improvement with an increase of 2 bits to the existing bit.

PART – B (5 x 3 = 15 MARKS)

11.	Check whether the following system is linear and time varying y(n) = nx(n).

12.	Define circular convolution.  How can one obtain linear convolution using it?

13.	Explain briefly frequency sampling structures and Lattice structure.

14.	Give the steps involved in the design of linear phase FIR filter by the frequency sampling method.

15.	Plot the truncation error for sign magnitude and two’s complement numbers.

PART – C (5 x 15 = 75 MARKS)

16.	The impulse response of an LTI System is h(n)  = {1, 2, 1, -1}.Find the response of system to an input signal  x(n) = {1,-1}.

(OR)

17.	Determine the frequency response He(jw) for the system and plot the magnitude response and phase  response  y(n) + 1/4 y(n-1) = x(n) –x(n-1).

18.	Compute the 8 point DFT of sequence x(n) = {1/2,1/2,1/2,1/2,0,0,0,0} using the inplace radix 2 DIF FFT algorithm.

(OR)

19.	State and prove Parseval’s theorem.

[P.T.O]

20.	Design a chebyshev type 1 band reject filter with the following specifications.  Pass band dc to 275 Hz and 2kHz  to infinity, stop band 550Hz to 1000Hz.  αp = 1 dB,  αs = 15 dB, F =  8kHz.

(OR)

21.	Using the bilinear transform design a high pass filter,monotonic in passband with cutoff frequency of 1000 Hz and down 10 dB at 350 Hz. The sampling frequency is 5000 Hz.

22.	Design an ideal differentiator with frequency response Hejω=jω ; −π≤ω≤π using Rectangular window.

(OR)

23.	Design a FIR filter approximating the ideal frequency response.

Hd(ejω)=e−jαω for ω≤π6

Determine the filter coefficients for N = 13.

24.	Explain the characteristics of a limit cycle oscillation with respect to the system described by the difference equation   y(n) = 0.95 y(n-1) + x(n).       Determine the dead band of the filter.

(OR)

25.	Find the effect of coefficient quantization on pole locations of the given second order IIR system when it is realized in direct form 1 and in cascade form.  Assume a word length of 4 bits through truncation.

H(z) = 11−0.9 z−1+0.2z−1