Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title : 	FLUID MECHANICS AND MACHINERY				      Time : 3 hours

Subject Code:	CE212		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State Pascal’s law.

2.	Define kinematic viscosity.

3.	Define co- efficient of friction.

4.	Write down the value of Reynolds number for laminar flow.

5.	Name the parts in venturimeter.

6.	Classify the losses in pipes.

7.	No slip condition is applicable to __________.

8.	What is the maximum theoretical suction head possible for a centrifugal pump?

9.	What is meant by surge tank?

10.	What is the use of draft tube in a turbine?

PART – B  (5 x 3 = 15 MARKS)

11.	Express 3m of water head in cm of mercury and pressure in kPa.

12.	State the equation of continuity in three dimensional flow.

13.	Name the important three types of Notches.

14.	Define Slip of reciprocating pump. When does the negative slip occur?

15.	Classify turbines according to flow.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the following terms:

a.	Mass density				b.	Weight density

c.	Specific volume			d.	Specific gravity

(OR)

17.	Explain the different types of mechanical pressure gauges.

18.	Derive the differential form equation of continuity for three dimensional flow and explain the details.

(OR)

19.	Derive Bernoulli’s equation from Euler’s equation of motion.

20.	In a 100mm diameter horizontal pipe a venturimeter of 0.5 contraction ratio has been fixed.  The head of water on the meter when there is no flow is 3 m(gauge).  Find the rate of flow for which the throat pressure will be 2 meters water absolute. The co-efficient of meter is 0.97. Take atmospheric pressure head = 10.3 m of water.

(OR)

21.	Draw the sectional view of Pitot’s tube and explain how the velocity of fluid flow is measured.

22.	Enumerate the working principle of a single acting reciprocating pump and explain its indicator diagram.

(OR)

[P.T.O]

23.	State the working of a centrifugal pump, draw its velocity diagram of impeller vane.

24.	Explain the working principle of Kaplan turbine with neat sketch.

(OR)

25.	Explain the working principle of Francis turbine with neat sketch.