Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	MECHANICS OF DEFORMABLE BODIES - II 			       Time: 3 hours

Subject Code:	CE241		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define deflection in a beam.

2.	What is the slope at fixed end of a cantilever beam ‘l’ meter long carrying point load w at its free end?

3.	State the effective length of a column with both ends being fixed.

4.	Long and short columns are failing by __________ and __________ failures respectively.

5.	Define thick shell.

6.	Theory suitable for brittle material failure is___________.

7.	For an I section which is symmetrical about XX and YY axes, the shear centre lies at the ________ of web.

8.	The theory of curved beam was postulated by __________.

9.	Shear centre is also known as __________.

10.	The product of inertia of a section about its principal axis is equal to __________.

PART – B (5 x 3 = 15 MARKS)

11.	Find the deflection at free end of a cantilever 4m long carrying an UDL of 6.5 kN/m if

EI = 63x 108 Nmm2.

12.	State the secant formula of estimating stress in columns loaded with eccentric loading.

13.	List out the assumptions made in the thick cylinder–Lame’s Theory.

14.	What is meant by shear centre?

15.	State the conditions for the unsymmetrical bending.

PART – C (5 x 15 = 75 MARKS)

16.	A cantilever 2 m long and having rectangular cross section of 100 mm wide and 200 mm depth is carrying an UDL of 4 kN/m for a length of 1 m at its free end. Find the slope and deflection. Take E = 190 kN/mm2.

(OR)

17.	A beam of length 6 m is simply supported at its ends and carries two point loads 48 kN and 40 kN at a distance of 1 m and 3 m respectively from the left support. Taking E as 200 kN/mm2 and I as 20 x 107 mm4, find the deflection under each load.

18.	A hollow cast iron column 200 mm external diameter and 150 mm internal diameter, 8 m long has both ends fixed. It is subjected to a compressive load. Taking a factor of as 6, fc = 560 N/mm2, and a = 1/1600, determine the safe Rankine load.

(OR)

19.	A column of 14 m high and having I section containing web of 1000 mm X 20 mm, flanges of 300 mm X 50 mm is with fixed ends at both the ends. Find out the safe load, the column can carry. Adopt a factor of safety of 4 and E = 210 kN/m3.

[P.T.O]

20.	A compound cylinder is made by shrinking one tube on another, the final dimensions being external diameter = 250 mm, internal diameter = 150 mm and diameter at the junction of tubes = 225 mm. If the radial pressure on the external surface of inner tube at the common radius 112.5 mm is 28 N/mm2, determine the maximum tensile stress in the metal of the outer tube.

(OR)

21.	A shaft is subjected to a maximum torque of 10 kNm and a maximum bending moment of   7.5kNm at a particular section. If the allowable equivalent stress in simple tension is 160N/mm2 find the diameter of the shaft according to maximum shear stress theory.

22.	a.	Derive the expression for shear centre of channel section.							(10)

b.	Determine the shear centre for a channel having web of 1800 mm x 10 mm and flanges of 1000 mm x 20 mm.															(5)

(OR)

23.	A steel ring has a rectangular cross section, 75 mm in the radial direction and 45 mm perpendicular to the radial direction. If the man radius of the ring is 150 mm and maximum tensile stress is limited to 180 N/mm2 calculate the tensile load the ring can carry.

24.	A beam of T-section with flange of 100 mm x 20 mm and web of 150 mm x 10 mm is 2.5 m long and simply supported at its ends. It carries a load of 3.2 kN inclined at 20 to the vertical and passing through the centroid of the section. If E = 200 kN/mm2 calculate the maximum tensile and maximum compressive stresses in the section.

(OR)

25.	A beam of T-section with flange of 100 mm x 20 mm and web of 150 mm x 10 mm is 2.5 m long and simply supported at its ends. It carries a load of 3.2 kN inclined at 20 to the vertical and passing through the centroid of the section. If E = 200 kN/mm2 calculate the maximum deflection and position of the neutral axis.