Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	APPLIED HYDRAULICS AND FLUID MACHINES 		       Time: 3 hours

Subject Code:	CE246		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Flow having no variation in the flow characteristics with the time variation is called as __________ steady flow.

2.	State the flow classification based on Froude Number.

3.	Retardation in the boundary layer is due to __________ force.

4.	Name the types of drags.

5.	What is relative velocity of jet?

6.	State the impulse momentum equation.

7.	What are turbines?

8.	Classify turbines.

9.	State the advantages of centrifugal pumps over reciprocating pumps.

10.	What is meant by slip?

PART – B (5 x 3 = 15 MARKS)

11.	What is meant by most economical section?

12.	What are the factors affecting boundary layer thickness?

13.	Define and differentiate radial flow and tangential flow.

14.	Define mechanical efficiency of a turbine.

15.	What is priming?

PART – C (5 x 15 = 75 MARKS)

16.	a.	Derive the condition for a most economical rectangular section.					(10)

b.	Discuss the channel transitions.												(5)

(OR)

17.	a.	A horizontal rectangular channel 4m wide carries a discharge of 16 m3/sec.  Determine 	whether a jump may occur at an initial depth of 0.5 m or not. If a jump occurs, determine 	the sequent depth to this initial depth.  Also determine the energy loss in the jump.	(7.5)

b.	Explain the formation of hydraulic jump.					           				(7.5)

18.	Explain the methods of controlling boundary layer.

(OR)

19.	a.	A submarine which may be supposed to approximate a cylinder 3 m in diameter and 15m 	long travels submerged at 1.54 m/s in sea water at 4o C.  Find the drag exerted on it.  Take 	the kinematic viscosity for sea water at 4 o C as 1.67 x 10-6 m2/s and density of sea water as    	1025 kg/m3.  Also find the friction drag.											(10)

b.	Draw and explain the approximate flow pattern and the pressure distribution around a flat 	plate placed perpendicularly in a stream flow.									(5)

[P.T.O]

20.	a.	A jet of water 75 mm diameter having a velocity of 20 m/s, strikes normally a flat smooth 	plate.  Determine the thrust on the plate (i) if the plate is at rest, (ii) if the plate is moving 	the same direction as the jet with a velocity of 5 m/s.  Also find the work done per second 	on the plate in each case and the efficiency of the jet when the plate is moving.		(10)

b.	Derive the expression for the force exerted by a jet of water on an inclined fixed plate in the direction of jet.															(5)

(OR)

21.	A jet of water having a velocity of 45 m/s impinges without shock a series of vanes moving at 15 m/s, the direction of motion of the vanes being inclined at 20o to that of the jet.  The relative velocity at outlet is 0.9 of that at inlet, and the absolute velocity of the water at exit is to be normal to motion of the vanes.  Find (a) vane angles at entrance and exit; (b) work done on vanes per unit weight of water supplied by the jet; and (c) the hydraulic efficiency.

22.	a.	Describe the working principle of Pelton wheel with a neat sketch.					(10)

b.	Describe the governing mechanism of Francis turbine.							(5)

(OR)

23.	a.	Design a Pelton wheel for a head of 80m and speed 300 rpm.  The Pelton wheel develops 	shaft power of 103 kW.  Take Cv = 0.98, speed ratio = 0.45 and overall efficiency as 80%. 																				(10)

b.	A turbine is to be operated under a head of 30m at 300 rpm.  The power generated is    2700 kW.  Find the specific speed of turbine.									(5)

24.	Draw a diagram showing the components of a centrifugal pump and explain the functioning of it.

(OR)

25.	Describe the principle and working of a reciprocating pump with neat sketches showing components and fitted with air vessels.