Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	MECHANICS OF SOILS 								       Time: 3 hours

Subject Code:	CE247		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define degree of saturation.

2.	Define void ratio.

3.	Define permeability.

4.	What is meant by piping failure?

5.	Define compaction.

6.	What is meant by pre consolidation pressure?

7.	State the uses of Newmark’s influence chart.

8.	What is meant by pressure bulb?

9.	Name the various types of slope failure.

10.	Name the various tests used to determine the shear strength of soil.

PART – B (5 x 3 = 15 MARKS)

11.	Write a short note on importance of Geotechnical Engineering.

12.	What do you understand by quick sand phenomena?

13.	What are the various factors that affect compaction?

14.	What are the various causes for settlement?

15.	What are the various factors that affect shear strength of soil?

PART – C (5 x 15 = 75 MARKS)

16.	A moist soil sample weighs 3.52 N. After drying in an oven, its weight is reduced to 2.9 N. The specific gravity of solids and mass specific gravity are respectively 2.65 and 1.85. Determine the water content, voids ratio and degree of saturation. Take unit weight of water = 10 kN/m3 .

(OR)

17.	Write a detailed note on classification of soils as per BIS.

18.	Explain the different methods for determination of coefficient of permeability in laboratory.

(OR)

19.	Explain the electrical analogy method for obtaining flow net.

20.	Write a detailed note on various field compaction methods.

(OR)

21.	a.	A clay layer 4m thick is subjected to a pressure of 55 kN/m2. If the layer has a double 	drainage and undergoes 50% consolidation in one year, determine the coefficient of 	consolidation. Take TV = 0.196.												(5)

b.	Explain any one method for determination of coefficient of consolidation. 			(10)

[P.T.O]

22.	The water table in a deposit of sand 8m thick is at a depth of 3m below the surface. Above the water table sand is saturated with capillary water. The bulk density of sand is 19.62 kN/m3. Calculate the effective pressure at 1m, 3m, and 8m below the surface. Plot the variation of total pressure, neutral pressure and effective pressure over the depth of 8m.

(OR)

23.	a.	A concentrated load of 2000 kN is applied at the ground surface. Determine the vertical 	stress at a point P which is 6m directly below the load. Also calculate the vertical stress at 	a point R which is at a depth of 6m but at a horizontal distance of 5m from the axis of the 	load.                                                                                							(10)

b.	Write a short note on contact pressure on sand.                          					(5)

24.	Discuss the friction circle method for stability analysis of slopes.

(OR)

25.	The following results were obtained from a series of consolidated undrained tests on a soil in which the pore water pressure was not determined. Determine the cohesion intercept and angle of shearing resistance.

| Sample No. | Confining pressure kN/m2 | Deviator stress at failure kN/m2 |
| --- | --- | --- |
| 1 | 100 | 600 |
| 2 | 200 | 750 |
| 3 | 300 | 870 |