Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	REINFORCED CONCRETE STRUCTURES - II 			       Time: 3 hours

Subject Code:	CE250		       						               	       Maximum Marks: 100

(Use of IS 456, IS 3370, IRC codes and Pigeaud’s charts is permitted)

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is a flight?

2.	When is the design of shear key necessary?

3.	Write any one method of analysis of rectangular tanks.

4.	Mention the grade of concrete which is used in the construction of water tank.

5.

What is the load for a wheeled vehicle as per IRC class AA loading?

6.	Write any one method used for the distribution of loads among the longitudinal girders.

7.	The method used for analysis of frame buildings against wind loads is __________.

8.	The capacity of the building materials, systems or structures to absorb energy by deforming in the inelastic range is called __________.

9.	When the absolute maximum bending moment due to a moving udl longer than the span of a simply supported beam occurs?

10.	What is the direction of yield line in one way slab?

PART – B (5 x 3 = 15 MARKS)

11.	What are the forces acting on a retaining wall?

12.	Find out the diameter of a circular tank which is having a flexible base for capacity of 200000 liters. The depth of water is to be 4m, including a free board of 200mm.

13.	List the various loads and forces to be considered in the design of bridge members.

14.	What is Substitute frame method? List the assumptions made in this method.

15.	What are the characteristic features of yield lines?

PART – C (5 x 15 = 75 MARKS)

16.	Design one of the flights of stairs of a school building spanning between landing beams to suit the following data:

Type of staircase = Waist slab type

Number of steps in flight= 12

Tread	T = 300 mm

Riser	R = 160 mm

Width of landing beams = 400 mm

Materials: M20 concrete and Fe415 HYSD bars

(OR)

17.	Design a cantilever retaining wall to retain an earth embankment 4m high above ground level. The density of earth is 18 kN/m3 and its angle of repose is 30 degrees. The embankment is horizontal at top. The safe bearing capacity of the soil may be taken as 200 kN/m2 and the coefficient of friction between soil and concrete is 0.5. Adopt M20 grade concrete and Fe 415 HYSD bars.

[P.T.O]

18.	A rectangular RC water tank with an open top is required to store 80,000 litres of water. The inside dimensions of the tank may be taken as 6 m x 4 m. The tank rests on all the four sides. Design the side walls of the tank using M20 grade concrete and Fe 415 HYSD bars.

(OR)

19.	Design an RCC circular water tank resting on ground with a flexible base and a spherical dome for storing 500,000 litres of water. The depth of storage is to be 4m. Freeboard = 200 mm. Adopt M20 grade concrete and Fe 415 HYSD bars. Permissible stresses should comply with the values recommended in IS 3370 and IS 456 codes.

20.	Determine the maximum live load bending moment in the extrerior girder of the T beam and slab with the following data for the two trains of IRC Class A loading using Courbon’s method.

Clear width of roadway = 7.5 m

Span(centre to centre of bearings) = 16 m

Average thickness of wearing coat = 80 mm

Concrete mix = M25 grade

Steel = Fe415 HYSD bars

(OR)

21.	Explain in detail the following: (i) Class AA loading, (ii) Class A loading, (iii) Class B loading, 	and (iv) Class 70 R loading.

22.	Explain Portal and Cantilever methods of analysis.

(OR)

23.	a.	Explain the substitute frame analysis for frames with neat sketches.					(10)

b.	List the measures in mitigating effects of earthquake in RC structures.				(5)

24.	Design a simply supported square slab of 5m side length to support a service load of 4 kN/m2. Adopt M20 grade concrete and Fe 415 grade HYSD bars. Assume load factors according to the IS 456: 2000 standard code.

(OR)

25.	Design a rectangular slab 6m x 4m in size and simply supported at the edges for a service load of 4 kN/m2. Assume coefficient of orthotrophy as 0.7, M20 grade concrete and Fe 415 HYSD bars.