Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	STRUCTURAL ANALYSIS - I 							       Time: 3 hours

Subject Code:	CE254		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	How to determine the statistical indeterminacy of a structure?

2.	State Betti Maxwell’s law.

3.	What are the principles of Muller Breslau?

4.	Is truss a beam?

5.	What is the condition to be considered for shallow arches?

6.	Give examples of arch action.

7.	Write the maximum sag or dip of the cable with respect to the horizontal span.

8.	What is the function of supporting trusses?

9.	What is meant by stress due to lack of fit?

10.	Define degrees of freedom.

PART – B (5 x 3 = 15 MARKS)

11.	Find the horizontal displacement at D of frame ABCD with length L and assume the flexural rigidity of the beam. EI to be constant throughout the member. Neglect strain energy due to axial deformation.

P

C

D

A

B

12.	What do you understand by equivalent uniformly distributed load?

13.	A 3 hinged arch of span 40m and rise 8m carries concentrated laods of 200kN and 150 kN at distances 0f 8m and 16m from the left end and an UDL of 50kN/m on the right half of the span. Find the horizontal thrust.

14.	What is suspension bridge? What are elements of the same?

15.	Why are compatibility conditions required?

PART – C (5 x 15 = 75 MARKS)

16. 	A continuous beam ABC of span 11m is subjected to an UDL of 30 kN/m and point load of 40 kN at 6m and at 8m. Analyse by the principle of superposition and determine the reactions.

(OR)

17.	A frame of ABCDE in which AB= 2m, BC=5m, CD=4m, CE= 2m and DE= 2m.  BC is subjected to the UDL of 20 kN/m and E has displacement of 0.4m. in which 30 kN point load is acting. Analyse by using any energy method. Sketch the bending moment diagram. EI- constant.

[P.T.O]

18.	Using Muller Breslau principle, draw the influence line for bending moment at the midpoint D of span AB  of the continuous beam ABC of span 15m in which AB = 9m, BC = 6m and AD=4.5m.  Determine the influence line ordinates at suitable intervals and plot them.

(OR)

19.	Using the relevant influence line diagram, find maximum bending moment and the maximum positive and negative shears at 4m from the left support A of a simply supported girder of span 10m, when a train of 4 wheel loads of 10 kN, 15 kN, 30 kN and 30 kN spaced at 2m, 3m, and 3m respectively across the span with the 10kN load leading.

20.	A symmetrical three hinged parabolic arch of span 40m and rise 8m carries an UDL of 30 kN/m over the half of the span. The hinges are provided at the supports and at the centre of the arch. Calculate the reactions at the supports.  Also calculate the bending moments, radial shear and normal thrust at a distance of 10m from the left support.

(OR)

21.	 A three hinged circular arch of span 16m and rise m is subjected to point loads of 100 kN and 80 kN at the left and right quarter span points respectively. Find the reactions at supports. Find also the bending moment, radial shear and normal thrust at 6m from left support.

22.	Find the forces in all the members of the pin jointed frame shown in Figure. All the members have the same area of cross section and Young’s modulus.

D

A

B

C

30

45

45

50kN

75kN

5

(OR)

23.	A suspension bridge of 100m span has two three hinged stiffening girders supported by two cables with a central dip of 10m. If three points loads of 20 kN each are placed along the centre line of the road way at 10, 15 and 20m from left hand hinge, find the shear force and bending moment in each girders at 30m from each end. Calculate the maximum tension in the cable.

24.	Draw the bending moment diagram of a continuous beam ABC shown in figure by three moment equation. The support B settles by 5mm below A and C. Also calculate reactions at A, B and C. Assume E= 200 GPa. I= 8x106mm4.

4m

2m

2m

4kN

A

B

C

(OR)

[P.T.O]

25.	A continuous beam ABCD is supported on springs at supports B and C as shown in figure. The loading is also shown in figure. The stiffness of springs is kb=EI/20 and kc=EI/30. Evaluate support reactions and draw bending moment diagrams. Assume EI to be constant.

4 kN/m

C

D

A

B

10 kN

8 kN

4m

4m

3m

1m