Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	STRUCTURAL ANALYSIS - II 							       Time: 3 hours

Subject Code:	CE255		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Write down the general condition of equilibrium at any joint of a continuous beam.

2.	Can slope deflection method be used to analyze statically determinate structures?

3.	What is the stiffness factor for a beam fixed at one end and freely supported at the other end?

4.	If right end of a fixed beam settles by , what is the moment developed at both ends?

5.	Explain the term transformation matrix.

6.	Define: Flexibility

7.	Write the stiffness matrix for a simply supported beam element.

8.	Stiffness matrix method is also known as ___________ method.

9.	Define: Dynamic load.

10.	What do you mean by steady state vibration?

PART – B (5 x 3 = 15 MARKS)

11.	Write the equilibrium equation used in slope deflection method.

12.	What do you mean by carry over factor?

13.	Define: Static and Kinematic Indeterminacy of a structure.

14.	What are the advantages of matrix methods over other classical methods of structural analysis?

15.	List out the forces associated with a basic dynamic system.

PART – C (5 x 15 = 75 MARKS)

16. 	A continuous beam ABC consists of spans AB = 3 m, BC = 4 m, the ends A and C being fixed. The spans AB and BC carry UDL 4 kN/m and 5 kN/m respectively. Find the support moments and draw the BMD using Slope Deflection Method. The beam is uniform in cross section throughout.

(OR)

17.	Analyse the continuous beam by Slope Deflection Method.

[P.T.O]

18.	A continuous beam ABC consists of two spans AB = 4m, BC = 3m, the end A being fixed, B and C simply supported as shown below. Analyse the beam and draw the BMD by Moment Distribution method.

(OR)

19.	A two span continuous beam ABC, has span AB = 6m. BC = 4 m, A and C being the end supports. Span AB carries a concentrated load 100 kN at a distance 3 m from A, Span BC carries a concentrated load 80 kN at a distance 1 m from B. Supports A, B and C are simply supported. IAB = 2I, IBC = I. Analyze the beam by Moment Distribution method and draw the BMD.

20.	Write briefly all the steps and equations involved in analysis of a pin jointed space frame by flexibility method.

(OR)

21.	Analyse the continuous beam shown below by flexibility method. EI is constant

22.	Develop the stiffness matrix for the beam element with respect to the 2 co-ordinates shown in

(OR)

23.	Analyse the continuous beam shown below by Stiffness Method. Determine the support moments.

24.	Briefly explain:	a.	Free Vibration 	b.	Forced Vibration 		c.	Degrees of freedom

(OR)

25.	Differentiate between

a.	Static Problem and Dynamic Problem 	b.	Damped and Undamped Vibration