Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	MECHANICS OF FLUIDS 								       Time: 3 hours

Subject Code:	CE277		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Classify the different types of fluids.

2.	State Pascal’s law.

3.	Flow between parallel plates of infinite extent is an example of _________.

4.	The concept of stream function which is based on the principle of continuity is applicable to________.

5.	What is a notch?

6.	Loss of head due to sudden enlargement is_________.

7.	The flow is said to be turbulent when Reynolds number is _________.

8.	When the flow is laminar where will be the maximum velocity occurs in a circular pipe?

9.	_________ is the square root of the ratio of the inertia force to the pressure force.

10.	Define Froude number.

PART – B (5 x 3 = 15 MARKS)

11.	Distinguish between ideal and real fluid.

12.	Explain the streak line flow of fluid.

13.	What are the different types of orifices?

14.	Differentiate between laminar and turbulent flow.

15.	State the uses of dimensional homogeneity.

PART – C (5 x 15 = 75 MARKS)

16.	Explain with neat sketches the Newton’s law of viscosity, dynamic and kinematic viscosity.

(OR)

17.	Derive an expression for the total pressure and centre of pressure for a vertically immersed surface.

18.	Derive the differential form equation of continuity for three dimensional flow and explain the details.

(OR)

19.	Derive Bernoulli’s equation from Euler’s equation of motion.

20.	A horizontal venturimeter with inlet diameter 200 mm and throat diameter 100mm is employed to measure the flow of water. The reading of the differential manometer connected to the inlet is 180 mm of mercury.  If the co-efficient of discharge is 0.98 determine the rate of flow.

(OR)

21.	A rectangular orifice 1.5 m wide and 1 m deep is discharging water from a tank. If the tank is 3.0 m above the top edge of the orifice, find the discharge through the orifice.

[P.T.O]

22.	a.	The diameter of a water pipe is suddenly enlarged from 350mm to 700mm. the rate of			flow through it is 0.25 cubic meter per second.  Calculate the loss of head in enlargement.

(7)

b.	Compare the rate of growth of the laminar boundary layer over a smooth flat plate placed in the water stream flowing at 2 m/s and placed in an air stream flowing at 2 m/s. Given that densities of water and air are 1000 and 1.2 kg/m3 respectively and the viscosities of water and air are 0.001 and 0.000019Ns/sq.m respectively. 						(8)

(OR)

23.	Derive an equation for loss of head due to friction in pipe.

24.	Classify the hydraulic models and explain.

(OR)

25.	In the model test of a spillway the discharge and velocity of flow over the model were 2.5 m3 / s and 1.5 m / s respectively.   Calculate the velocity and discharge over the prototype which is 36 times the model size.