Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	SURVEYING 											       Time: 3 hours

Subject Code:	CE280		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define surveying.

2.	Define contour.

3.	Define latitude.

4.	What is meant by transiting of theodolite?

5.	State the advantage of using anallactic lens.

6.	Define orientation in plane table survey.

7.	State the relationship between radius and degree of curve.

8.	Where will you provide a transition curve?

9.	Classify triangulation system.

10.	What is meant by trigonometrical leveling?

PART – B (5 x 3 = 15 MARKS)

11.	Compare the various methods for calculation of reduced level.

12.	State the desired relationship between fundamental lines of theodolite.

13.	State the advantages and disadvantages of plane table survey.

14.	Calculate the length of long chord and mid ordinate for a circular curve of radius 100m and deflection angle 30.

15.	Write a detailed note on signals commonly used in surveying.

PART – C (5 x 15 = 75 MARKS)

16.	What are the different methods of contouring? Explain any two methods in detail with sketches.

(OR)

17.	The following offsets were taken at 15m interval from a survey line to an irregular boundary line 	3.50, 4.30, 6.75, 5.25, 7.50, 8.80, 7.90, 6.40, 4.40, 3.25m.  Calculate the area enclosed between the survey line, irregular boundary line and the first and last offset by trapezoidal and Simpsons rule.

18.	Explain in detail how you will measure horizontal and vertical angle using a theodolite.

(OR)

19.	The record of a closed traverse is given below with two distances missing

| Line | Length (m) | Bearing |
| --- | --- | --- |
| AB | 100.5 | N 30 30 E |
| BC | ---- | S 45 E |
| CD | 75 | S 40 30 W |
| DE | 50.5 | S 60  W |
| EA | --- | N 40 15 W |

Calculate the length BC and EA.

[P.T.O]

20.	A tacheometer was set up at a station C and the following readings were obtained as a staff held vertically. Instrument constants are 100 and 0.15.R.L. of B.M. is 750.5 m.

| Instrument Station | Staff Station | Vertical angle | Hair readings |
| --- | --- | --- | --- |
| C | B.M | - 5 20 | 1.15,1.8,2.45 m |
| C | D | 8 12 | 0.75, 1.5,2.25 m |

Calculate horizontal distance CD and R.L. of D.

(OR)

21.	Explain radiation, intersection and traversing method of plane tabling.

22.	Explain the various surveys to be conducted for the location of a highway project.

(OR)

23.	Two tangents intersect at a chainage of 1250m. The angle of intersection is 150. Calculate all the necessary data for setting out the circular curve of radius 250m by Rankines method.  Assume peg interval as 20m.

24.	In a trignometrical measurement of the difference in level of two stations P and Q, 10480 m apart, the following data were obtained.

Instrument at P, angle of elevation of Q = 0’ 15”

Instrument at Q, angle of depression of P = 3’ 33”

Height of instrument at P = 1.42 m.

Height of instrument at Q = 1.45 m.

Height of signal at P = 3.95 m.

Height of signal at Q = 3.92 m.

Find the difference in level between P and Q

Take R sin 1” = 30.38m.

(OR)

25.	Write a detailed note on corrections for base line measurement with formulas.