Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	CHEMICAL REACTION ENGINEERING 				       Time: 3 hours

Subject Code:	CL205		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the unit of the rate of reaction based on the unit volume of fluid?

2.	Distinguish between elementary and non-elementary reactions.

3.	Classify chemical reactor based on the mode of operation.

4.	Define space velocity.

5.	What is axial dispersion coefficient?

6.	Draw the E and F curve for ideal mixed flow.

7.	What is series reaction?

8.	What is meant by diffusion poison?

9.	Define bimodal pore size distribution.

10.	Define film conversion parameter.

PART – B (5 x 3 = 15 MARKS)

11.	Define zero order reaction.

12. 	What is meant by semi-batch reactor?

13.	Explain about RTD function.

14.	Explain briefly about Gas-Solid reactions with the size of the particle shrunk.

15. 	Define surface diffusion.

PART – C (5 x 15 = 75 MARKS)

16. 	Explain the methods for the determination of order of reaction.

(OR)

17. 	a.	Show that for a first order reaction

– ln (1-XA) = k.t

b.	Show that the for nth order reaction, ratio of time required for completion of reaction to time required for 75% completion of reaction is independent of the concentration and is function of n alone.

18.	Derive the rate expression for the first order irreversible gaseous reaction taking place in a plug flow reactor.

(OR)

19. 	How will you compare the performance of a CSTR and a PFR for a first order reaction?

20. 	Explain the RTD in a) Plug flow reactor	b)	Mixed flow reactor

(OR)

[P.T.O]

21. 	A liquid phase first order reaction A → R is carried out in a plug flow reactor. It is desired to have 98% conversion. The flow characteristics of the reactor are studied by introducing a tracer in the reactor. The tracer concentration in the exit stream is monitored as a function of time and the data is tabulated below. Calculate the reaction rate constant and the dispersion.

| T min | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 10 | 15 | 20 | 30 | 41 | 52 | 67 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tracer Concentration g/m3 | 9 | 57 | 81 | 90 | 90 | 86 | 77 | 67 | 47 | 32 | 15 | 7 | 3 | 1 |

22. 	 Explain about the unreacted core model for spherical particles of unchanging size.

(OR)

23. 	A moving grate is fed with a feed having different size fractions. The reacting gas flows cross current to feed. The maximum residence time of feed over the moving grate is 10 minutes. The feed consists of 25% of 50 m radius particles, 25 % of 80 m radius particles, 10 % of 100 m radius particles, 15 % of 150 m radius particles and 25 % of 200  radius particles. The time required for complete conversion of these different size particles is 4, 8, 12, 15 and 18 minutes respectively. Calculate the fraction of solids converted on the moving grate reactor.

24. 	 Derive the effectiveness factor of cylindrical catalyst pellet with suitable diagram.

(OR)

25. 	 Write in detail about Slurry Reactors.