Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	DIGITAL ELECTRONICS 								       Time: 3 hours

Subject Code:	EC209		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is the advantage of gray code?

2.	State Demorgan laws.

3.	Write the truth table of NAND gate.

4.	What is decoder?

5.	Write the characteristic equation of JK flip flop.

6.	Differentiate between synchronous and asynchronous counters.

7.	What is state reduction?

8.	Define race condition.

9.	What is the difference between PLA and PAL?

10.	Define fan out.

PART – B (5 x 3 = 15 MARKS)

11.	Convert the hexadecimal number 68BE to binary and then convert it to octal.

12.	Draw the logic diagram of half subtractor and write the truth table.

13.	Draw the diagram of 4 bit ring counter assuming initial value is 1000. Also draw the timing signals.

14.	What is hazard? Explain its types.

15.	Specify the values of fan out, propagation delay and power dissipation of TTL NAND gate.

PART – C (5 x 15 = 75 MARKS)

16.	Simplify the following Boolean function into SOP and POS.  F(A,B,C,D)= ∑ (0,1,2,5,8,9,10). Also draw the logic diagram using AND and OR gates.

(OR)

17.	Minimize the following function using QM method. f(x1, x2, x3, x4)= ∑ (0,5,7,8,9,10,11,14,15)

18.	Implement BCD to excess3 converter using truth table and K-map.

(OR)

19.	Implement F(A,B,C,D)= ∑ (1,3,4,11,12,13,14,15) using multiplexer.

20.	Explain 4 bit synchronous binary counter with neat diagram.

(OR)

21.	Draw the diagram of 4 bit universal shift register and explain.

22.	Design a circuit that detects a sequence of 3 (or) more consecutive 1’s in a string of bits coming through input line. Use JK flip flop for implementation.

(OR)

23.	Design a 3 bit binary counter using T flip flop.

[P.T.O]

24.	Implement the following using PLA.

F1 (A,B,C)= ∑ (0,1,2,4)

F2 (A,B,C)= ∑ (0,5,6,7)

F3 (A,B,C)= ∑ (0,3,5,7)

(OR)

25.	Briefly explain TTL and ECL logic families.