Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title: 	MICROPROCESSORS AND APPLICATIONS		       Time: 3 hours

Subject Code:	EC238		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Write down the 8085 instructions used for serial communication.

2.	How many T-states are necessary to perform the following 8085 instruction:  MVI B, 40H

3.	Give the maximum addressing capacity of 8086.

4.	Mention the usefulness of the following instruction in 8086:  LEA

5.	Which bit of 8255 control word differentiates I/O mode and BSR mode?

6.	What is the use of HRQ signal in DMA controller?

7.	What is the function of interrupt mask register (IMR) in 8259?

8.	What are the three types of counters available in 8275?

9.	Define absolute decoding.

10.	What is settling time in D/A converters?

PART – B (5 x 3 = 15 MARKS)

11.	Classify the addressing modes in 8085 with examples.

12.	Differentiate the minimum and maximum mode of 8086.

13.	Define PPI. What are the modes of operation supported in 8255 PPI?

14.	What are the primary features of 8275 CRT controller?

15.	Discuss the various options available for interfacing high power devices.

PART – C (5 x 15 = 75 MARKS)

16.	Draw the functional block diagram of microprocessor 8085 and explain in detail.  Also explain the significance of pin signals with pin diagram.

(OR)

17.	a. 	Draw the timing diagram for fetching and execution on 8085 processor for the instruction MVI A, 	30H.																		(8)

b. 	Write an ALP in 8055 to find the negative numbers in a block of data.					(7)

18.	Explain with example, the different groups of instructions in 8086.

(OR)

19.	a. 	Explain the register structure of 8086.

b. 	Discuss the various addressing modes of 8086.

20.	Draw the functional diagram of programmable DMA controller and explain the operation.

(OR)

21.	Explain the various parallel data transfer schemes.

22.	Draw the block diagram of programmable interval timer and discuss the various features of it in detail.

(OR)

23.	Give the block diagram of interfacing the CRT using 8275 CRT controller and explain how the characters are produced on the screen.

24.	With the help of block diagram, explain the approach to control the speed of DC motor.

(OR)

25.	Describe with necessary diagram, how an 8 bit DAC is interfaced to 8085.