Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	DIGITAL ELECTRONICS  								       Time: 3 hours

Subject Code:	EC284		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Convert (111000101)2  to octal number.

2.	Find the 9’s complement of (70042)10.

3.	What is binary decoder?

4.	________ and ________ are the universal gates.

5.	Flip flop is a ________.

6.	Define rise time.

7.	What is a sequential circuit?

8.	Define state table.

9.	Define Fan-out.

10.	What are the types of TTL logic?

PART – B (5 x 3 = 15 MARKS)

11.	Convert the (3A.2F)16 to decimal number and (A72E)16 to octal number.

12.	Write a short note on parity generator/checker.

13.	Define shift registers. Mention its types.

14.	Give the comparison between synchronous and asynchronous sequential circuits.

15.	State the advantages and disadvantages of TTL.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Do the following operations

i)	Convert (6ABC.2A)16  to decimal number									(5)

ii)	Find the 2’s complement of 1101 and 1111001								(5)

b.	Apply Demorgan’s theorem to the following expression(A+B+C)D.				(5)

(OR)

17.	Simplify the expression using Karnaugh map Y= π (0, 1, 4, 5, 6, 8, 9, 12, 13, 14)

18.	a.	Implement F(A, B, C, D)= (1, 3, 4, 11, 12, 13, 14, 15) using multiplexer.			(10)

b.	Explain the function of half adder and full adder circuits with neat logic diagrams and truth 	tables.																		(5)

(OR)

19.	Design a binary to BCD code converter using unused combinations as don’t care conditions.

20.	Draw and explain the logic diagrams of JK and T flip flops. Also state their characteristics and truth tables.

(OR)

21.	a.	Design Mod 4 synchronous counter using JK flip flop and implement it.			(10)

b.	Give the comparison between synchronous and asynchronous counters.				(5)

[P.T.O]

22.	Draw and explain the block diagram of synchronous mealy and moore machines.

(OR)

23.	Explain the Sequential Logic circuit partitioning with one example. Also explain the rules for partitioning.

24.	Draw a TTL circuit with totem pole output and explain its working.

(OR)

25.	a.	Draw the basic CMOS inverter and explain its operation.							(8)

b.	Explain the Integrated Injection Logic (IIL) with neat logic diagram.				(7)