Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	NETWORK THEORY 									       Time: 3 hours

Subject Code:	EE246		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	In a three phase system the volt ampere rating is given by __________.

2.	Two wattmeter method of power measurement can be used to measure power in__________ circuits.

3.	__________ is the property associated with two or more coils.

4.	__________ tuned circuits are generally used in Radio receivers.

5.	The time constant of a series R-L circuit is __________.

6.	The system is said to be stable only if __________.

7.	Name the different types of wave filter.

8.	A two port network is simply a network inside a black box, and the network has only __________ of accessible terminals.

9.	Hurwitz polynomial possesses one of the condition that __________.

10.	In the first foster form the presence of first element Capacitor Co indicates __________.

PART – B (5 x 3 = 15 MARKS)

11.	Explain the advantages of three phase system.

12.	Briefly explain the concept of coefficient of coupling.

13.	Derive the DC response of R-L circuit.

14.	What are the classifications of attenuators? Explain.

15.	List out the properties of positive real function.

PART – C (5 x 15 = 75 MARKS)

16.	A Three phase balanced delta connected load of (4+j8)Ω is connected across a 400 V, 3 	phase balanced supply. Determine the phase currents, line currents. Assume the phase 	sequence to be RYB. Also calculate power drawn by the load.

(OR)

17.	Discuss the three phase Power measurement by two wattmeter method.

18.	What is ideal transformer? Discuss in detail.

(OR)

19.	What are tuned circuits? Derive the expression for obtaining the output voltage and maximum amplification.

20.	Derive the expression for the current i(t) if series R-L circuit is excited by sinusoidal input.

(OR)

21.	What are poles and zeros? Discuss the necessary conditions for driving point function and transfer function in a given network.

[P.T.O]

22.	Write short notes on

a.	Cascade connection of two port network. 										(5)

b.	Series connection of two port network.                                                                   		(5)

c.	Parallel connection of two port network.                                                                   	(5)

(OR)

23.	Design a m-derived low pass filter (both T and π section) having cut-off frequency of 1 kHz, design impedance of 400Ω, and the resonant frequency 1100Hz.

24.	a.	Explain in detail the property of Hurwitz polynomials with suitable example.              (8)

b.	Discuss the frequency response of reactive one –ports.                                                  (7)

(OR)

25.	The driving point impedance of a one port reactive network is given by 						Z(s) =  5(s2+4)(s2+25)s(s2+16).  Obtain the first and second Foster networks.