Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	ELECTRICAL MACHINE DESIGN 						       Time: 3 hours

Subject Code:	EE252		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	List out few conducting materials used in the design of electrical machines.

2.	Name the materials used to decrease the value of permeability.

3.	For a simple lap winding, the winding pitch is equal to _________.

4.	The commutator segment in the DC machine is equal to _________.

5.	What is the percentage of silicon used to make the transformer stampings?

6.	Which component of the no load current of the transformer is opposite in phase to the induced emf?

7.	In case of induction motors the value of air gap flux density usually taken is _________.

8.	Which type of slots are generally used in induction motors?

9.	Write the range of Short circuit ratio of turbo alternators.

10.	When is the hunting sound produced in the synchronous motor?

PART – B (5 x 3 = 15 MARKS)

11.	Write the fundamental requirements of good insulating materials.

12.	Draw the diagram of Commutator and explain its segments.

13.	Describe the necessity of cooling tubes in the transformer.

14.	Enumerate the importance of air gap design in the induction motor.

15.	Describe the applications of damper windings.

PART – C (5 x 15 = 75 MARKS)

16. 	Discuss about various duties and ratings of Rotating Machines and give their respective temperature - time curves.

(OR)

17.	A field coil has a heat dissipating surface of 0.15 m2 and length of mean turn 1 m. It dissipates loss of 150 W, the emissivity being 34 W/m2_°C. Estimate the final steady temperature rise of the coil and its time constant if the cross section of the coil is     100*50 mm2. Specific heat of copper is 390 J/kg° C. The space factor is 0.56. Copper weighs 8900 kg/m3.

18.	a.	Explain the effects of choice of number of poles in a DC Machine on

i)	Frequency of flux reversal		ii)	Weight of iron	iii)	Weight of copper and

(iv)	Length of commutator.                                                         	                  		(8)

b.	A 5 kW, 250 V, 4 pole, 1500 rpm DC Shunt Generator is designed to have a square pole face. The specific magnetic loading and specific electric loadings are 0.42 Wb/m2 and 15000 ac/m respectively. Find the main dimensions of the machine. Assume full load efficiency= 0.87 and pole arc to pole pitch ratio is 0.66.                                    		(7)

(OR)

[P.T.O]

19.	a.	Discuss various methods to determine mmf required for teeth of an Electric Machine.	(8)

b.	Determine the apparent flux density in teeth of a DC machine if the real flux density in teeth is 2.15 Wb/m2, slot pitch is 28 mm, slot width is 10 mm, gross core length is 0.35 m, no. of ventilating ducts is 4, each 10 mm wide. Magnetizing force corresponding to flux density of 2.15 Wb/m2 is 55000 AT/m and iron stacking factor is 0.9.    				(7)

20.	a.	Derive the output equation of a three phase transformer. 							(8)

b.	The ratio of flux to full load mmf in a 400 kVA, 50 Hz single phase core type transformer is 2.4*10–6. Calculate the net iron area and the window area of the transformer if the maximum flux density in the core is 1.3 Wb/m2, Current density is    2.7 A/mm2 and window space factor is 0.26. Also calculate the full load mmf. 						(7)

(OR)

21.	Obtain the equivalent circuit components of the transformer and explain the no load current calculation in detail.

22.	Determine the main dimensions, number of radial ventilating ducts, number of stator slots and turns per phase of a 3.7 kW, three phase, 400 V, 4 Pole, 50 Hz squirrel cage Induction Motor to be started by a Star-Delta starter. Given that the average flux density in the air gap = 0.45 Wb/m2; Ampere Conductor per meter of armature periphery = 23000, full load efficiency = 0.85, full load power factor = 0.84 and kw = 0.955. Take L/τ = 1.5.

(OR)

23.	Discuss the step by step procedure to design the rotor of a squirrel cage Induction Motor.

24.	Briefly describe the designing procedure of field winding of the turbo alternator.

(OR)

25.	a.	Derive the output equation of an AC machine. 									(8)

b.	Determine the main dimensions of a 100 kVA, 50 Hz, three phase 375 rpm alternator. The average air gap flux density is 0.55 Wb/m2 and ampere conductors per metre is 28000. Given that L/τ must be between 1 to 5. The maximum permissible peripheral speed is 50 m/sec. The runaway speed is 1.8 times synchronous speed. 						(7)