Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	POWER ELECTRONICS 								       Time: 3 hours

Subject Code:	EE254		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	General purpose diodes have relatively high recovery time, in the order of _________.

2.	MOSFET is a _________controlled device.

3.	The angle at which the SCR is turned off is called _________.

4.	Mention the two types of commutation.

5.	Another name of the variable frequency system is _________.

6.	Duty ratio is the ratio of _________.

7.	A VSI is one in which the DC source has _________ impedance.

8.	Symmetrical pulse width modulation is another name for _________.

9.	Mention one application of UPS.

10.	Size of conductors in DC transmission can be reduced as there is no _________.

PART – B (5 x 3 = 15 MARKS)

11.	Compare MOSFET and IGBT.

12.	Describe the function of freewheeling diode.

13.	Define current limit control.

14.	Define three methods of voltage control available in single phase inverters.

15.	Write briefly about types of HVDC link

PART – C (5 x 15 = 75 MARKS)

16.	a.	Describe the steady state characteristics of power BJT.							(7)

b.	Draw and explain the two transistor model of a thyristor.							(8)

(OR)

17.	a.	Draw and explain the reverse recovery characteristics of a power diode.				(7)

b.	Give the basic structure, characteristics and explain working principle of IGBT.		(8)

18.	Describe the operation of a single-phase, two-pulse, mid-point converter with relevant voltage and current waveforms. Discuss how each SCR is subjected to a reverse voltage equal to double the supply voltage in case turns ratio from primary to each secondary is unity.

(OR)

19.	What is a dual converter? With neat circuit and waveforms explain the operation of three phase dual converter in circulating current mode. Derive necessary equations for operation on load and No load.

20.	What is cyclo converter? Describe the basic principle of working of a single-phase to single-phase, step down cyclo-converter for both continuous and discontinuous conduction.

(OR)

[P.T.O]

21.	a.	With required circuit diagram, waveforms and equations, explain the step-up chopper.(7)

b.	For a type- A chopper dc source voltage is 230V, load resistance is 10Ω. Take a voltage drop of 2 Volt across the chopper when it is ON. For a duty cycle of 0.4 calculate (i) average and RMS value of output voltage (ii) chopper efficiency					(8)

22.	Discuss the principle of working of a three-phase bridge inverter with an appropriate circuit diagram. Draw phase and line voltage waveforms on the assumption that each thyristor conducts for 120 and the resistive load is star-connected. The sequence of firing of various SCR’s should also be indicated in the diagram.

(OR)

23.	a.	How is harmonic reduction achieved in stepped – wave inverter?   					(7)

b.	What is Pulse Width Modulation? List the various PWM techniques. How do they differ from each other?																(8)

24.	a.	Discuss about the power converter control using microprocessor and microcontrollers.

(7)

b.	Explain the application of phase controlled rectifiers for DC motor drives with necessary circuit.

(OR)

25.	a.	Discuss the generation of control signals for single phase AC to DC converters.		(7)

b.	Explain the principle of bipolar HVDC system.									(8)