Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	POWER SYSTEM ANALYSIS 							       Time: 3 hours

Subject Code:	EE255		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are the two types of disturbances?

2.	Define per unit value.

3.	Define primitive network

4.	What is a bus?

5.	What is load flow solution?

6.	What is a tap changing transformer?

7.	What is meant by a fault?

8.	Write the relative frequency of occurrence of line-to-line fault.

9.	How is the system stated as a stability criterion at stable or unstable?

10.	Mention one assumption in an stability.

PART – B (5 x 3 = 15 MARKS)

11.	Define swing curve. What is the use of swing curve?

12.	What are the different types of constraints used in economic dispatch problem?

13.	Why power flow analysis is made?

14.	Write the procedure to compute an element of bus admittance matrix.

15.	Explain the requirements of planning the operation of a power system.

PART – C (5 x 15 = 75 MARKS)

16.	What is the need for base values?	List the various advantages of per-unit computations.

(OR)

17.	a.	A generator rated at 30MVA, 11 kVA has a reactance of 20%. Calculate its per unit 	reactance for a base of 50 MVA and 10 kV.										(8)

b.	The base kV and base MVA of a 3-phase transmission line is 33 kV and 10 MVA respectively.	 Calculate the base current and base impedance. 						(7)

18.	Explain the modeling of generator, load, shunt capacitor and transmission line for short circuit, and power flow and stability studies.

(OR)

19.	Explain briefly about the formation of bus impedance matrix with an illustration.

20.	Explain clearly the algorithms steps for solving load flow equations using Newton-Raphson method when the system contains all types of buses.

(OR)

[P.T.O]

21. 	The system data for a load flow problem are given in table 1 and table 2.

a.	Compute Y bus

b.	Determine bus voltage at the end of 1st iteration by Gauss-Seidel method. Take acceleration factor as 1.6.

Table-1 Line data								Table-2 Bus data

| Buscode | P Demand | Q Demand | V, p.u | Remarks |
| --- | --- | --- | --- | --- |
| 1 | - | - | 1.06 <0 | Slack |
| 2 | 0.5 | 0.2 | - | PQ |
| 3 | 0.4 | 0.3 | - | PQ |

| Buscode of lines | Admittance(p.u) |
| --- | --- |
| 1-2 | 2-j8 |
| 1-3 | 1-j4 |
| 2-3 | 0.6-j2.6 |

22.	Explain the various constraints in unit commitment problem with suitable examples.

(OR)

23.	Derive the coordination equation for transmission loss in economic scheduling problem.

24.	Draw a flow chart which shows the various steps involved in modified Euler’s method to solve the swing equation.

(OR)

25.	 Derive the power-angle equation for a synchronous machine.