Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	POWER SYSTEM PROTECTION AND SWITCHGEARS      Time: 3 hours

Subject Code:	EE256		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is an overvoltage?

2.	What are the causes of overvoltages?

3.	What is voltage surge?

4.	Define impulse ratio.

5.	What is reliability?

6.	What is a time setting multiplier?

7.	Name one common scheme used for protection against faults.

8.	What are the different protections provided for AC motor?

9.	What is a circuit breaker?

10.	Define rated frequency.

PART – B (5 x 3 = 15 MARKS)

11.	How are the circuit breakers rated?

12.	Explain insulation failure.

13.	What are the types of circuit breakers?

14.	What are the faults that occur in rotor circuits?

15.	What is earthing?

PART – C (5 x 15 = 75 MARKS)

16.	Give the block diagram of a typical microprocessor based protective relay for motor protection.

(OR)

17.	Explain the principle of a distance relay, stating clearly the difference between electromagnetic version of an impedance relay and mho relay.

18.	Derive an expression for restriking voltage and rate of restriking voltage (RRRV) in terms of system voltage, inductance up to the fault location and bushing to earth capacitance of the circuit breaker.

(OR)

19.	Define and explain the following terms as applied to circuit breakers a) arc voltage b) restriking Voltage	c) recovery voltage.

20.	What is lightning? Describe the mechanism of lightning discharge.

(OR)

21.	Discuss the construction, principle and working of a valve type arrester.

22.	Explain the following:		a.	Earthing screen	b.	Overhead ground wires 			(8+7)

(OR)

23.	Describe solid grounding and resistance grounding for earthing the neutral point of generator.

[P.T.O]

24.	Write short notes on nature and cause of faults.

(OR)

25.	With a neat sketch explain the Merz-Price circulating current scheme for protection of alternators.