Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	LINEAR INTEGRATED CIRCUITS 					       Time: 3 hours

Subject Code:	EE258		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define open loop gain.

2.	The effect of change in temperature of an op-amp varies with __________,	__________ and __________.

3.	List any two applications of Integrator.

4.	What is a Notch filter?

5.	Draw the diagram of a Zero crossing detector.

6.	List any two multiplier techniques.

7.	__________ is the basic principle of switching regulators.

8.	Pulse width of a Monostable Multivibrator is controlled by __________ components.

9.	Define resolution of ADC and DAC.

10.	An 8 bit successive approximation ADC is driven by a 1 MHz clock. Find its conversion time.

PART – B (5 x 3 = 15 MARKS)

11.	Define CMRR and Slew rate.

12.	Derive the expression of closed loop voltage gain of Non-Inverting amplifier.

13.	What is a Logarithmic amplifier?

14.	Mention the difference between linear regulators and switching regulators.

15.	What are the draw backs of binary weighted resistor DAC?

PART – C (5 x 15 = 75 MARKS)

16.	Draw the functional block diagram of an Op-amp and explain in detail each block.

(OR)

17.	Explain the methods of frequency compensation used in operational amplifiers.

18.	Explain in detail an active op-amp differentiator and analyze the practical differentiator and its frequency response.

(OR)

19.	Design a second order Low pass Butterworth filter having cut off frequency 1 kHz.

20.	a.	With the help of neat diagram and waveform explain the full wave precision rectifier 	circuit.																		(8)

b.	Write short notes on Multiplier and Divider circuits.								(7)

(OR)

21.	Write short notes on:	a.	Triangular Wave generator		b.	Norton amplifiers.

22.	Explain with a neat sketch, the working of IC555 as an Astable multivibrator and derive the expression for its duty cycle.

(OR)

23.	a.	With block schematic explain the working principle of PLL.

b.	How is PLL used as frequency multiplier?

[P.T.O]

24.	Explain the dual slope A/D converter with neat circuit diagram and explain how it provides noise rejection.

(OR)

25.	For a 4 bit R-2R ladder network determine the size of each step if R = 10kΩ, RF = 40kΩ and Vcc = ±15V. Calculate the output voltage for D0 = 1, D1= 0 D2 = 1  D3 = 1 if bit 1 applied as 5V and bit 0 is applied as 0V.