Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	MEASUREMENTS AND INSTRUMENTATION 			       Time: 3 hours

Subject Code:	EE262		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The unit for power is _________.

2.	Damping torque in D’arsonval Galvanometer is due to _________.

3.	In electrodynamometer type wattmeters, current coils designed for carrying heavy currents are laminated to reduce _________.

4.	What is the meter constant of an energy meter?

5.	Kelvin’s Double Bridge is used to measure_________.

6.	Guard circuits are used to eliminate the errors caused by _________.

7.	What is active transducer?

8.	Give the principle of strain gauge.

9.	What is strip chart recorder?

10.	What is spectrum analyzer?

PART – B (5 x 3 = 15 MARKS)

11.	Three resistors have the following ratings:  R1= 37 Ω ±5%, R2 = 75 Ω ± 5%, R3= 50 Ω ±5%.  	Determine the magnitude and limiting error in Ohm and percentage of limiting error for series 	combination.

12.	Give the advantages and disadvantages of moving iron power factor meters.

13.	Give the merits and demerits of Maxwell’s bridge.

14.	What is Piezo-electric effect?

15.	What is Total Harmonic Distortion?

PART – C (5 x 15 = 75 MARKS)

16.	Explain with a neat sketch the constructional features of Permanent Magnet Moving Coil Instruments.

(OR)

17.	Discuss the following of current transformer in detail.

a.	Effect of PF on secondary winding 												(4)

b.	Effect of change in primary winding current										(4)

c.	Effect of change in secondary winding current 									(4)

d.	Effect of change of frequency													(3)

18.	With a neat circuit and phasor diagrams, explain measurement of power using instrument transformers.

(OR)

19.	Elaborate on the construction details of induction type energy meters.

20.	a.	Explain in detail the Wheatstone bridge.											(8)

b.	With a neat diagram, explain in detail about Hay’s bridge.							(7)

(OR)

[P.T.O]

21.	Discuss the use of Schering Bridge in measurement of capacitance.

22.	Explain Linear Variable Differential Transformer with its circuits and give its advantages and disadvantages.

(OR)

23.	a.	Explain the construction details of thermistors and discuss resistance temperature 	characteristics of thermistors.													(8)

b.	Describe the measurement of temperature with thermocouple.						(7)

24.	Discuss about any two types of digital voltmeters in detail.

(OR)

25.	Explain with a neat sketch, the operation of dual channel oscilloscopes in detail.