Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	HIGH VOLTAGE ENGINEERING 						       Time: 3 hours

Subject Code:	EE271		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	The energy required to knock out an electron from a fermi level is known as the________ function.

2.	At normal temperature and pressure, the gases are excellent ________.

3.	The condition Veαd =________defines the threshold sparking condition.

4.	________law doesn’t hold good for many gaseous mixtures.

5.	Electrostatic generators are used for generating ________.

6.	What is Impulse voltage?

7.	The unit of specific dielectric loss is ________.

8.	________ effect is used to measure very high direct current.

9.	What are the two types of tests required for high voltage testing of electrical equipment?

10.	What is Wet flash over test?

PART – B (5 x 3 = 15 MARKS)

11.	What is secondary emission?

12.	What is penning effect?

13.	What is multistage generator?

14.	List the nondestructive testing methods for insulation faults.

15.	List the various types of overhead line insulators.

PART – C (5 x 15 = 75 MARKS)

16.	Discuss and derive the Townsend’s first ionization coefficient for electron produced in the direction of field.

(OR)

17.	What is Ionization? Explain all the types of ionization.

18.	Briefly explain the Townsend's criterion for break down.

(OR)

19.	Briefly explain all the types of breakdown mechanism in solid dielectrics.

20.	Draw neatly and explain the principle and operation of Vande graaf generator.

(OR)

21.	Explain the principle and operation of producing high ac voltages using cascaded transformer.

22.	With a neat sketch explain the principle and working of Electrostatic voltmeters.

(OR)

23.	Explain the measurement techniques used for the following

a.	Resistivity		b.	Large capacitance										(7+8)

24.	Explain in detail the various tests carried out in testing of Circuit breakers.

(OR)

25.	Explain in detail the various tests carried out in testing of Bushings.