Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title : 	ELECTRIC DRIVES AND CONTROL					      Time : 3 hours

Subject Code:	EE273		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by electric drive?

2.	What are the different types of electric drive?

3.	Mention the methods to control the speed of DC shunt motor.

4.	What is discontinuous conduction mode?

5.	What are the applications of ac drive?

6.	What is meant by stator voltage control?

7.	Whether synchronous motor is self starting or not?

8.	Define step angle.

9.	What is microcontroller?

10.	Write the disadvantage of digital control.

PART – B  (5 x 3 = 15 MARKS)

11.	Mention the advantages of Electric drive.

12.	What do you understand by constant torque drive and constant speed drive?

13.	What are features of variable frequency control?

14.	Differentiate voltage and current source inverter.

15.	Which drive is suitable for lift? Justify your choice.

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss the speed-torque characteristics of various electric drives.

(OR)

17.	What is load equalization? How does a flywheel tend to equalize the load on the driving motor?

18.	With suitable diagram explain the operation of single phase full converter fed dc drive at discontinuous mode and derive the speed equation.

(OR)

19.	In a single quadrant chopper the supply voltage V = 220V. The chopper supplies a dc motor load with back emf of 22V. The motor has a resistance R = 0.25Ω and L = 1mH. The on time is 1msec. while T = 2.5 msec. Calculate the average output current, the maximum and minimum values of load current.

20.	Discuss the stator frequency control.

(OR)

21.	With neat sketch explain the static Kramer system.

22.	Describe using a circuit the self controlled mode of operation of a synchronous motor.

(OR)

23.	Explain the principle of operation and control of stepper motor.

24.	With neat block diagram explain the microprocessor based control of Electric drive.

(OR)

25.	Discuss the selection of drive for (a) paper mill (b) cement mill.