Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	NETWORK ANALYSIS AND SYNTHESIS 				       Time: 3 hours

Subject Code:	EE295		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	The inverse transform of 1/S is _______.

2.	The driving point impedance is defined as the ratio of _______.

3.	The function is said to be having simple poles and zeros only if _______are not repeated.

4.	The time constant of a series R-L circuit is _______.

5.	The number of branches in a tree is _______ than the number of branches in a graph.

6.	The cut set schedule gives the relation between _______.

7.	Hybrid parameters find extensive use in _______ circuits.

8.	The attenuation of a wave filter can be expressed in _______.

9.	The roots of the odd and even parts of a Hurwitz polynomial P(s) lie on the _______.

10.	In the first Foster form, the presence of first element capacitor Co indicates pole at _______.

PART – B (5 x 3 = 15 MARKS)

11.	Write the steps to obtain residues by pole-zero plots.

12.	Compare time domain and frequency domain analysis.

13.	Define (a) Tree (b) Links (c) Co-tree of the graph.

14.	Discuss the classification of filters.

15.	Write any three properties of Hurwitz polynomials.

PART – C (5 x 15 = 75 MARKS)

16.	a.	Describe the restrictions on location of poles and zeros in driving point functions.	(8)

b.	Verify whether the following expression for driving point impedance is suitable for 	representing a passive one port network. Z(s)= s2+s+2s4+5s3+6s2 .					(7)

(OR)

17.	Describe in detail the steps to derive time domain response from pole-zero plots.

18.	Explain in detail about Amplitude and Phase Characteristics from pole zero plot with example.

(OR)

19.	a.	Discuss about sinusoidal response of R-L circuit.									(8)

b.	Explain about Complex Loci for a RC circuit in detail.							(7)

a

b

c

d

1

2

3

20.	a.	For the given graph in figure, write the tie set schedule and obtain the relation between 	branch currents and link currents.												(8)

b.	Discuss about duality and construction of a dual network in detail.					(7)

(OR)

[P.T.O]

1

2

3

4

a

b

c

d

e

21.	a.	For the given graph in figure, write the cut set schedule and obtain the relation between 	tree branch voltages and branch voltages.										(8)

b.	Describe the formulation of equilibrium equation based on loop.					(7)

22.	a.	Find the Z parameters for the circuit shown in Figure.								(8)

a1

a2

b1

b2

v1

v2

I1

I2

1

2

2

4

b.	The impedance parameters of a two port network are Z11= 6Ω; Z22= 4Ω;  Z12= Z21= 3Ω. 	Compute the Y parameters.													(7)

(OR)

23.	a.	Derive equations for ZOT, ZOC, ZSC and propagation constant of a T-Network filter.	(8)

b.	Design a m-derived high pass filter with a cut off frequency of 10 kHz; design impedance of 500Ω and m = 0.4															(7)

24.	a.	Check whether the given polynomial is Hurwitz or not.

P(s) =2s5  + 3s4 + 6s3 + 5s2 + 3s + 4												(8)

b.	Find the first foster form of the driving point function of Z(s)= .			(7)

(OR)

25.	a.	Check whether the function F(s) =  is positive real function or not.			(8)

b.	Find the first Cauer form of the function Z(S)=.							(7)