Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	ELECTRIC CIRCUIT ANALYSIS 						      Time: 3 hours

Subject Code:	EI250		       						               	       Maximum Marks: 100

#### All resistance are in  and all capacitance are in F unless specified

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Draw the V – I characteristics of a Pure Inductor.

2.	State Ohms Law.

3.	The power factor of series RLC circuit at resonance condition is _________.

4.	Under steady state condition a capacitor behaves like a

a.	Open circuit    b.	Short circuit   	c.	Inductor    	d.	Resistor

5.	If a network contains B branches and N nodes, then the number of mesh current equations would be _________.

6.	What does the value of K (co-efficient of coupling) indicates?

7.	Give the condition for maximum power transfer.

8.	Thevenin’s circuit is _________ of the Norton’s equivalent.

9.	A first order circuit with a single resistor, if the resistor value is doubled, the time constant will be _________ for an RC−circuit.

10.	The complete response of a network excited from dc sources is the sum of _________ response and _________ response.

PART – B (5 x 3 = 15 MARKS)

11.	Find all branch currents in the network shown below

12.	A coil is represented by a series combination of L = 50mH and R = 15Ω.  Calculate the quality factor at a) 10kHz , b) 50kHz.

13.	For the coupled circuit shown below, find the ratio V2 / V1 which results in zero current I1

I1

I2

V2

14.	State Compensation theorem and explain in brief where this can be applied.

15.	Obtain an expression for transient current in a series RC circuit when it is suddenly excited by a DC source.

[P.T.O]

PART – C (5 x 15 = 75 MARKS)

16.	What is the voltage across A and B in the circuit shown below

A

B

(OR)

17.	Determine the current delivered by the source in the circuit shown below

2

2

2

2

2

2

1

1

4

30  V

+

–

18.	a.	Two impedances Z1 = 20 + j 10 and Z2 = 10 – j 30 are connected in parallel and this 	combination is connected in series with Z3 = 30 + j X.  Find the value of X which will 	produce resonance.								               					(10)

b.	A three – phase, three – wire, Δ- connected load yields wattmeter readings of 1154W and 577W.  Obtain the load impedance if the line voltage is 141.4V.                     		(5)

(OR)

19.	For the circuit shown in figure below, determine the total impedance, total current and phase angle.

20.	Use node voltage method to obtain the current I in the network shown below.

500V

5090V

(OR)

[P.T.O]

21.	In the network shown below, determine voltage V which results in zero current through 2+ j3Ω impedance by mesh analysis

30V

22.	Using compensation theorem, determine the ammeter reading when it is connected to 6Ω resistor in the circuit shown below.  The internal resistance of ammeter is 2Ω.

(OR)

23.	Replace the following circuit by a single voltage source in series with a resistor as seen from 	resistor R.  Use source transformation and obtain the Norton’s equivalent.

24.	Obtain an expression for the transient current in a R – L circuit when excited by a sinusoidal source given by v = Vcos(ωt+θ)

(OR)

25.	A series RLC circuit with R = 200Ω, L = 0.10H and C = 13.33μF, has an initial charge on the capacitor of Qo = 2.67x10–3 C.  A switch is closed at t= 0, allowing the capacitor to 	discharge.  Obtain the current transient