Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	SIGNALS AND SYSTEMS 								       Time: 3 hours

Subject Code:	IT240		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	Define Step function.

2.	Is the signal x(n) = sin 7n a odd or even signal?

3.	What is an LTI system?

4.	The condition to be satisfied for stability of LTI systems is __________.

5.	What are frequency shaping filters?

6.	The group delay at each frequency equals the __________ of the slope of the __________ at that frequency.

7.	What is interpolation?

8.	For a causal LTI system the ROC of the system function lies in _________.

9.	Find the Z-Transform of x(n) = u(n) – u(n-3).

10.	Give any two methods used to find inverse Z-Transform.

PART – B (5 x 3 = 15 MARKS)

11.	Find whether the following is a energy or power signal x (n) = (1/3)n u(n).

12.	What is convolution sum and convolution integral?

13.	What are the advantages of logarithmic frequency scale in continuous time systems?

14.	What is sampling theorem?  List the effects of under sampling.

15.	State the Parseval’s theorem in Z – Transform.

PART – C (5 x 15 = 75 MARKS)

16.	Test whether the following signals are periodic or not and if the signal is periodic, calculate the fundamental period.

a.	x (t) = 5 u (t) + 7 sin2t						b.	x (t) = 15 cos (10πt + π/6)

c.	x (n) = sin (πn/8) + 12 cos (πn/4 + π/3)		d.	x (n) = ej2π n/3   + ej3π n/4

e.	x (n) = 2 cos (10t +1) – sin (4t – 1)

(OR)

17.	Explain in detail about the basic properties of discrete time systems.

18.	Consider the following sequences and find the convolution sum of the two signals.

xn=1, 0≤n≤4

(OR)

19.	Discuss in detail about the causal LTI systems described by differential and difference equations.

20.	Explain in detail the Time-domain properties of ideal frequency selective filters.

(OR)

[P.T.O]

21.	a.	Analyze the time domain and frequency domain behavior of second order continuous time 	systems.																		(8)

b.	Plot the impulse response step response and the bode plot for various damping ratios.

(7)

22.	Consider an ideal High pass filter whose frequency response is specified as

Hjω=1, ω>ωc

a.	Determine the impulse response of the filter

b.	Determine S(0) and S (∞), where S(t) is the step response of the filter.

(OR)

23.	Explain in detail about Sampling of Discrete time systems.

24.	Find the Inverse Z-Transform of X(z)=1−13 z−11−z−1 1+2z−1;   |Z| > 2.

(OR)

25.	A causal LTI system is described by the difference equation y[n] = y[n-1] + y[n-2] + x[n-1]. Find the system transfer function and plot the poles and zeros of H(z) and indicate the ROC. Also find the unit sample response of the system.