Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	HEAT ENGINES AND FLUID MACHINERY 			       Time: 3 hours

Subject Code:	ME223		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What is the unit for kinematic viscosity?

2.	What is priming?

3.	Give the function of surge tank.

4.	Define the term Turbine.

5.	Sketch the P-V Diagram for Isothermal process.

6.	What is meant by the term Reversibility?

7.	Name the factors that affect air standard efficiency of Diesel cycle?

8.	What are all the modifications to be carried out for increasing the gas turbine efficiency?

9.	Write the Stefan –Boltzmann equation for heat transfer.

10.	What are the uses of fins in I.C engines?

PART – B (5 x 3 = 15 MARKS)

11.	State the Newton’s law of viscosity and give examples of its application.

12.	What is a Draft tube? Why is it used in a reaction turbine?

13.	An inventor claims to have developed an efficient heat engine which would have a heat source at 1000ºC and reject heat to a sink 50ºC and gives an efficiency 90% .  Justify whether the claim is possible.

14.	What are the assumptions made for air standard cycle analysis?

15.	Give three examples of heat transfer with internal heat generation.

PART – C (5 x 15 = 75 MARKS)

16.	a.	A differential manometer is connected at the two points A and B of two pipes as shown in 		figure.  The pipe A contains a liquid of sp. gravity = 1.5.While Pipe B contains a liquid of 		sp. gravity = 0.9.The pressures at A and B are 1 Kgf/cm2 and 1.80 Kgf/cm2 respectively. 			Find the differential mercury level in the differential Manometer.					(10)

b.	Calculate the capillary rise in a glass tube of 2.5mm dia when immersed vertically  in (a) 	Water and (b) Mercury. Take surface tension 0.0725N/m for water and 0.52 N/m for 	mercury in contact with air. The sp. gravity for mercury is given as 13.6 and angle of 	contact 130º.																	(5)

(OR)

[P.T.O]

17.	a.	Explain the working of a single stage centrifugal pump with a neat sketch. 			(10)

b.	What is an air vessel and give the function for reciprocating pump					(5)

18.	A Kaplan turbine develops 24647.6 kW power at an average head of 39 meters.   Assuming a speed ratio of 2, flow ratio of 0.6, diameter of the boss equal to 0.35 times the diameter of the runner and an overall efficiency of 90 %.  Calculate the diameter, speed and specific speed of the turbine.

(OR)

19.	A Pelton wheel is tube designed for the following specification Shaft power = 11772 kW, Head= 380 m, Speed = 750 rpm, Overall efficiency = 86 % Jet diameter is not to exceed 1/6 of wheel diameter. Determine: (a) Wheel Diameter   (b) No. of Jets required, and (c) Dia. of the jet. Take Kv1 = 0.985	Ku1 = 0.45

20.	1.5 kg certain gas at a pressure 8 bar and 20ºC occupies the volume of 0.5 m3. It expands adiabatically to a pressure of 0.9 bar and volume 0.73 m3.Determine the work done during the process, gas constant, ratio of specific heats, values of two specific heats, change in internal energy and change in enthalpy.

(OR)

21.	Derive an expression for thermal efficiency of a carnot cycle with P-V and T-S diagram.

22.	Sketch the P-V and T-S diagrams for the otto cycle and obtain an expression for its air standard efficiency.

(OR)

23.	A gas turbine working on theoretical air cycle draws air initially at 25ºC and 1 bar.  The maximum pressure and temperature is 3 bar and 650ºC.  Calculate air standard efficiency, Heat supplied, Heat rejected, work output/Kg of air and exhaust temperature.

(OR)

24.	a.	Hot air at 40ºC is flowing through a steel pipe of 10 cm diameter the pipe is covered with 	two layer insulating materials of thickness 4 cm and 3 cm and their corresponding thermal 	conductivities are 0.1 and 0.32 W/m.K. The inside and outside convective heat transfer 	coefficient are 50 W/m2 and 15 W/m2.The outer temperature is 10ºC.  Find the heat loss 	per 	meter length of steam.													 	(10)

b.	Differentiate between natural convection and forced convection.		          		(5)

(OR)

25.	Air at 25ºC flows at a speed of 7m/s over a plate and heated to 85ºC. Calculate the heat transfer coefficient at a distance of 20cm.