Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	ENGINEERING THERMODYNAMICS 					       Time: 3 hours

Subject Code:	ME228		       						               	       Maximum Marks: 100

[Usage of Steam Tables and Compressibility chart may be permitted]

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	What is intensive property?  Give examples.

2.	State Charles law of ideal gas.

3.	A Carnot cycle works between the temperatures of 700 K and 300 K. Find its efficiency?

4.	What is a heat engine?

5.	What is saturation pressure?

6.	State the steady flow energy equation.

7.	What is Graham’s law of diffusion?

8.	Write down the Vander Wall’s equation for real gases?

9.	What is the thermal efficiency of an Otto cycle having a compression ratio of 12?

10.	What is mean effective pressure?

PART – B (5 x 3 = 15 MARKS)

11.	Define the following:  Thermodynamic system, Enthalpy, Point function.

12.	Draw the Carnot cycle for a heat engine on PV and TS coordinates.

13.	Atmospheric air at 1 bar and 30C is compressed isothermally to a pressure of 10 bar.  Calculate the work required to compress the air?

14.	Determine the pressure of 5 kg of carbon-di-oxide contained in a vessel of 2m3 capacity at 27C considering it as perfect gas.

15.	Draw the PV and TS diagrams of a Diesel cycle.

PART – C (5 x 15 = 75 MARKS)

16.	Write down the steady state steady flow (SSSF) energy equation. Reduce the expression for the following thermal equipments: a.	Nozzle	b.	Turbine and 	c.	Condenser.

(OR)

17.	A piston cylinder contains air at 600 kPa, 290 K and a volume of 0.01 m3.  A constant pressure process gives 54 kJ of workout.  The internal energy change for this process is 2357 kJ/kgK.  Find the final temperature of the air and the heat transfer input.

18.	Prove the proposition of the II law that “It is impossible to construct an engine that operates between two given reservoirs and is more efficient than a reversible engine operating between the same two reservoirs” through a thought analogy.

(OR)

19.	A reversible heat engine operates between two reservoirs at 827 C and 27C.  Engine drives a Carnot refrigerator maintain - 13C and rejecting heat to reservoir at 27C.  Heat input to the engine is 2000 kJ and the network available is 3000 kJ.  How much heat is transferred to refrigerant and total heat rejected to reservoir at 27C?

[P.T.O]

20.	Derive the fundamental relation PV = C for an adiabatic process.  Also find the expression for work done in an adiabatic compression process.

(OR)

21.	A piston/cylinder arrangement shown in figure below initially contains air at 150kPa, 400C.  The setup is allowed to cool to the ambient temperature of 20C.

a. 	Is the piston resting on the stops in the final state?  What is the final pressure in the 	cylinder?

b.	What is the specific work done by the air during this process?  Draw the PV and TV 	diagrams.

22.	Write short notes on

a.	Real gas		b.	Avagadro’s law	c.	Gay-Lussac’s law

d.	Vander Wall’s equation of state 	e.	Lower calorific value.

(OR)

23.	With a neat sketch explain the working of Junker’s calorimeter.

24.	Draw the Otto cycle on P-V and T-S coordinates.  Derive the expression for the mean effective pressure of an Otto cycle.

(OR)

25.	An ideal air-standard Diesel cycle engine has a compression ratio of 18 and a cutoff ratio of 2.  At the beginning of the compression process, the working fluid is at 100 kPa, 27C (300 K).  Determine the temperature and pressure of the air at the end of each process, the network output per cycle [kJ/kg] and the thermal efficiency.