Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title : 	HEAT AND MASS TRANSFER							      Time : 3 hours

Subject Code:	ME230		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is thermal resistance?

2.	What is critical radius of insulation?

3.	What is meant by transient heat conduction?

4.	What is Biot number?

5.	Define forced convection.

6.	What is the other name of fins?

7.	What is counter flow heat exchanger?

8.	Define mass concentration.

9.	What is drop wise condensation?

10.	Define reflectivity.

PART – B  (5 x 3 = 15 MARKS)

11.	What is thermal diffusivity?

12.	What is a semi infinite body?

13.	Define thermal boundary layer.

14.	State Fick’s law of diffusion.

15.	Define Shape factor.

PART – C  (5 x 15 = 75 MARKS)

16.	A steel pipe line(K = 50 W/mK) of I.D 100 mm and O.D. 110mm is to be covered with two layers of insulation each having thickness of 50 mm.  The thermal conductivity of the first insulation material is 0.06 W/mK and that of the second is 0.12 W/mK.  Calculate the loss of heat per meter length of pipe and the interface temperatures between the two layers of insulation when the temperature of the inside tube surface is 250˚C and that of the outer surface of the insulation is 50˚C.

(OR)

17.	Derive heat conduction equation for heat flow in Cartesian coordinates.

18.	A 40 x 40 cm copper slab 5mm thick at a uniform temperature of 250˚C suddenly has its surface temperature lowered to 30˚C.  Find the time at which the slab temperature becomes 90˚C. Take C = 380J/KgK, K=370 W/mK, ρ = 9000 Kg/m3; h = 90 W/m2K.

(OR)

19.	A steel ball of radius 2.5 cm, initially at a temperature of 450˚C is suddenly placed in a controlled environment in which the temperature is maintained at 100˚C. Calculate the time required for the ball to attain a temperature of 150˚C. Take C = 460J/KgK, K= 35 W/mK,          ρ = 7800 Kg/m3; h = 10 W/m2K.

20.	An air stream at 0˚C is flowing along a heated plate at 90˚C at a speed of 75m/s.  The plate is 45 cm long and 60 cm wide.  Calculate the average values of friction, heat transfer coefficient and the rate of energy dissipation from the plate.

[P.T.O]

(OR)

21.	Calculate the coefficient of heat transfer by free convection between a horizontal wire and air at 25˚C. The surface of the wire is at 95˚C and its diameter is 2.5mm.  Also find the rate of heat loss per unit length of the wire.

22.	Hot oil with capacity rate of 2500 W/K flows through a double pipe heat exchanger. It enters at 360˚C and leaves at 300˚C.Cold fluid enters at 30˚C and leaves at 200˚C.  If the overall heat transfer coefficient is 800 W/m2K, determine the heat exchanger area required for (a) parallel flow and (b) counter flow.

(OR)

23.	Air at 25˚C and atmospheric pressure, containing small quantities of iodine flows with a velocity of 5m/s inside a 3 cm inner diameter tube.  Determine the mass transfer coefficient from the air stream to the wall surface.  Assume DAB(iodine-air) = 0.82 x 10-5 m2/s .

24.	A pipe carrying steam having an outside diameter of 25 cm runs in a large room and is exposed to air at a temperature of 30˚C. The pipe surface temperature is 410˚C.  Calculate the loss of heat to surroundings per meter length of pipe due to thermal radiation. The emissivity of pipe surface is 0.8.  What would be the loss of heat due to radiation if the pipe is enclosed in a 40cm diameter brick conduit of emissivity 0.90?

(OR)

25.	Water is boiled at a rate of 30 Kg/h in a copper pan, 30 cm in diameter, at atmospheric pressure.  Estimate the temperature of the bottom surface of the pan assuming nucleate boiling conditions.