Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	DESIGN OF MECHANICAL TRANSMISSION SYSTEMS     Time: 3 hours

Subject Code:	ME231		       						               	       Maximum Marks: 100

(Use of approved Design Data Book permitted, any missing data may be suitably assumed.)

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	___________ steel is widely used as a ball bearing material.

2.	V-belts are not so durable as flat ones, because of high _________ stresses.

3.	Gear materials should have ___________ and ___________.

4.	The number of chain links will be usually ___________.

5.	The teeth are ___________ shaped in zerol bevel gears.

6.	Stub tooth has ___________ addendum and dedendum than the standard tooth.

7.	For optimum design, the number of gears on the last shaft (spindle) should be the __________.

8.	The housings of reduction gears are usually made of ___________.

9.	___________ screw is used to obtain very fine movement.

10.	Ratchet and pawl arrangement is used mostly in ___________ machines.

PART – B (5 x 3 = 15 MARKS)

11.	Mention the modes of failure of rolling contact bearings.

12.	What is virtual number of teeth in helical gears?

13.	Sketch any two types of bevel gears.

14.	List the methods of lubrication in speed reducers.

15.	What are the parameters that influence the cam size?

PART – C (5 x 15 = 75 MARKS)

16.	Design a V-belt drive and calculate the actual belt tensions and average stress for the following data: D=500 mm, d=150 mm, C=925 mm, n1 = 1000 rpm,  n2 = 300 rpm and P= 7.5 kW.

(OR)

17.	a.	Explain the types of friction in sliding contact bearings. 							(8)

b.	Illustrate and explain the variation of coefficient of friction with bearing characteristic 	number. 																		(7)

18.	Design a chain drive to actuate a compressor from a 10 kW electric motor at 960 rpm. The compressor speed is to be 350 rpm. Minimum center distance should be 0.5 m. Motor is mounted on an auxiliary bed. Compressor is to work for 8 hours/day.

(OR)

19.	A general purpose enclosed gear train is based on parallel helical gears, specified life is 36,000 hours. Torque at driven shaft is 411 Nm. Driving shaft speed is 475 rpm. Velocity ratio is 4. It is desired to have standard center distance.  Design the helical gear pair.

[P.T.O]

20.	Design a pair of cast iron bevel gears for a special purpose machine tool to transmit 3.5 kW from a shaft at 500 rpm to another at 800 rpm. The gears overhang in their shafts. Life required is 8000 hours.

(OR)

21.	Design a worm gear drive to transmit 22.5 kW at a worm speed of 1440 rpm. Velocity ratio is 24:1. An efficiency of atleast 85% is desired. The temperature rise should be restricted to 400C. Determine the required cooling area.

22.	Design a 9-speed gear box for a drilling machine with speeds ranging from 30 and 1000   rpm. The unit is driven by a 7 kW, 2880 rpm motor.

a.	Sketch the layout of gear box. 													(3)

b.	Draw the speed diagram. 														(4)

c.	Calculate the number of teeth on all gears. 										(4)

d.	Determine the center distance and module of gears in first stage. 					(4)

(OR)

23.	A gear box is to be designed for the following specifications:

Power = 5 kW. Number of speeds = 12. The minimum and maximum speeds are 100 rpm and 1200 rpm. The 12 speeds are obtained as 2X2X3.

a.	Sketch the layout of the gear box.  												(4)

b.	Draw the speed diagram. 														(6)

c.	Estimate the number of teeth in the gears for the third stage. 						(5)

24.	Design a ratchet wheel for a winch. Data given: Braking torque = 150 Nm; Brake drum diameter = 350 mm. The allowable bending stress for the wheel is 30 N/mm2.

(OR)

25.	The displacement specifications of follower are given below:

Stroke of the follower 25 mm

Outstroke takes place with SHM during 90˚ of cam rotation

Return stroke takes place with SHM during 75˚ of cam rotation

Cam rotates with a uniform speed of 800 rpm

Determine the induced contact stress, assuming follower mass as 0.25 kg and spring stiffness as 100 N/mm, when the cam rotation is 15˚.Take the thickness of the plate cam as 10 mm. The base circle radius and the follower roller radius are 25 mm and 10 mm respectively. Material used is steel.