Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	MECHANICS OF MACHINES - II 						       Time: 3 hours

Subject Code:	ME255		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	State D’Alembert’s principle.

2.	What is meant by piston effort?

3.	Define co efficient of fluctuation of speed.

4.	What is the function of flywheel?

5.	What are the conditions of equilibrium of rotating masses?

6.	Give the expression for primary unbalanced force of reciprocating masses.

7.	Define critical speed.

8.	What is meant by damping factor?

9.	Give the expression for length of equivalent shaft.

10.	What is meant by torsional vibrations?

PART – B (5 x 3 = 15 MARKS)

11.	What are the forces acting on the reciprocating parts of an engine?

12.	Draw the turning moment diagram for a four stroke cycle internal combustion engine.

13.	What is meant by swaying couple and write expression for maximum and minimum value of swaying couple.

14.	Explain different types of vibrations.

15.	Derive the expression for natural frequency of free torsional vibrations.

PART – C (5 x 15 = 75 MARKS)

16.	The crank and connecting rod of a steam engine are 0.3 m and 1.5 m in length. The crank rotates at 180 rpm clockwise. Determine the velocity and the acceleration of the piston when the crank is at 40 degrees from the inner dead centre position. Also determine the position of the crank for zero acceleration of piston.

(OR)

17.	Derive the expression for displacement, velocity and acceleration of a slider crank mechanism.

18.	The turning moment diagram for a multi cylinder engine has been drawn to a scale 1 mm = 600 Nm vertically and 1 mm = 3 horizontally. The intercepted areas between the output torque curve and the mean resistance line, taken in order from one end, are as follows +52, -124,+92,  -140,+85,-72 and +107mm2 ,when the engine is running at a speed of 600 rpm. If the total fluctuation of speed is not to exceed ± 1.5% of the mean, find the necessary mass of the flywheel of radius 0.5 m.

(OR)

19.	The turning moment curve for an engine is represented by the equation T = (20000+9500sin2-5700cos2) Nm, where  is angle moved by the crank from the inner dead centre. If the resisting torque is constant, find (i) power developed by the engine (ii) Moment of Inertia of the flywheel in kg – m2, If the total fluctuation of speed is not to exceed 1 % of mean speed which is 180 rpm and (iii) angular acceleration of flywheel when the crank has turned through 45 from inner dead centre.

[P.T.O]

20.	A, B, C and D are four masses carried by a rotating shaft at radii 100,125,200 and 150 mm respectively. The planes in which the masses revolve are space 600 mm apart and the masses of B, C and D are 10 kg, 5 kg and 4 kg respectively. Find the required mass A and the relative angular setting of the four masses so that the shaft shall be in complete balance.

(OR)

21.	Explain the direct and reverse crank method for determining unbalanced forces in radial engines.

22.	The measurements on mechanical vibrating system shows that it has a mass of 8 kg and that the springs can be combined to give an equivalent spring of stiffness 5. 4 N/mm. If the vibrating systems have a dashpot attached which exerts a force of 40 N when the mass has a velocity of 1 m/s, find (a) critical damping coefficient (b) Damping factor (c) logarithmic decrement (d) the ratio of two consecutive amplitudes.

(OR)

23.	A body of mass 20 kg is suspended from a spring which deflects 15 mm under this load. Calculate the frequency of free vibrations and verify that a viscous damping force amounting to approximately 1000 N at a speed of 1 m/s is just sufficient to make the motion aperiodic. If when damped to this extent, the body is subjected to a disturbing force with a maximum value of 125 N making 8 cycles per second, find the amplitude of the ultimate motion.

24.	A steel shaft 1.5 m long is 95 mm in diameter for the first 0.6 m of its length, 60 mm in diameter for the next 0.5 m of the length and 50 mm in diameter for the remaining 0.4 m of its length. The shaft carries two flywheels at two ends, the first having a mass of 900 kg and 0.85 m radius of gyration located at the 95 mm diameter end and the second having a mass of 700 kg and 0.55 m radius of gyration located at the other end. Determine the location of the node and the natural frequency of free torsional vibration of the system. The modulus of rigidity of the shaft material is 80GN/m2.

(OR)

25.	Determine the natural frequencies of torsional oscillations for the following system. The system is a reciprocating IC engine coupled to a centrifugal pump through a pair of gears. The shaft from the flywheel of the engine to the gear wheel is of 60 mm diameter and 950 mm length. The shaft from the pinion to the pump is of 40 mm diameter and 300 mm length. The engine speed is 1/4th of the pump speed.

Moment of Inertia of the flywheel is 800 kg m2

Moment of Inertia of the gearwheel is 15 kg m2

Moment of Inertia of the pinion is 4kg m2

Moment of Inertia of the pump is 17 kg m2

The modulus of rigidity of the shaft material is 84 GN/mm2.