Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	FINITE ELEMENT ANALYSIS 							       Time: 3 hours

Subject Code:	ME269		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1. 	What are the phases of FEM?

2.	List the types of Boundary conditions.

3.	What is Element Assembly?

4.	Classify the types of element.

5.	What are local coordinates?

6.	What is a shape function?

7.	What are higher order elements?

8.	What is a serendipity element?

9.	Write the one dimensional stiffness matrix.

10.	Fluid mechanics is a study of ________ problems.

PART – B (5 x 3 = 15 MARKS)

11.	List the method of weight residual.

12.	What is Finite Element Analysis?

13.	Briefly discuss about coordinate systems in a shape function.

14.	What are the advantages of higher order element?

15.	In a plane strain model, consider x= 1.38 x 108 N/m2, y = - 6.89 x 107 N/m2, E = 2.068 x 1011 N/m2 and  = 0.3. Determine the value of normal stress z.

PART – C (5 x 15 = 75 MARKS)

16. 	Explain neatly with an equation of boundary and initial value problem.

(OR)

17.	Discuss about the applications of finite element analysis packages.

18.	Discuss the general procedure of FEA.

(OR)

19.	Deduce the stiffness matrix for the spar element used in FEA.

20.	Derive the shape functions for six noded isoparametric element using global coordinate system.

(OR)

21.	Derive the shape function for triangular element.

22.	The triangular element having node1(x=1,y=1), node2 (x=3,y=4) and node3 (x=2,y=7) with the displacement along x and y at nodal points are U1=0.001,U2=0.003,U3=-0.002, V1=-0.004, V2=0.002 and V3=0.005 respectively. Find the displacement U and V at point P(x=2, y=5) by considering polynomial equations.

(OR)

23.	Derive the serendipity shape function for eight-node quadrilateral element.

[P.T.O]

24.	Consider the bar in figure loaded as shown.  Determine the nodal displacement, element stress and support reactions.

400mm2

250mm2

E=200x109N/m2

150mm

150mm

P = 300 kN

300mm

150mm

150mm

300mm

(OR)

25.	A composite wall consists of three materials, as shown in Figure. The outer temperature is To = 20 ° C. Convection heat transfer takes place on the inner surface of the wall with L = 800°C and h = 25 W/m2. °c. Determine the temperature distribution in the wall