Reg. No. ________

Karunya University

(Karunya Institute of Technology and Sciences)

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

Supplementary Examination - June 2011

Subject Title:	METROLOGY AND COMPUTER AIDED INSPECTION	       Time: 3 hours

Subject Code:	ME281		       						               	       Maximum Marks: 100

#### Answer ALL questions

PART – A (10 x 1 = 10 MARKS)

1.	Define Accuracy.

2.	The concept of establishing a valid calibration of a measuring instrument or measurement standard, by step-by-step comparison with better standards up to an accepted or specified standard is called _________.

3.	The greatest care should be exercised in protecting the slip gauges and their case from _________, _________ and _________.

4.	State any two limitations of the Sine Bar.

5.	List the factors affecting surface roughness.

6.	Define Concentricity.

7.	State the two corrections to be applied for the measurement of effective diameter.

8.	Define Module.

9.	State the types of CMM.

10.	Which type of Optical Flats has both the surfaces flat and parallel to each other?

PART – B (5 x 3 = 15 MARKS)

11.	Define Sensitivity and Readability.

12.	What are the two categories of measurement for which Clinometers can be used?

13.	Compare Straightness test by using Spirit level and Auto Collimator.

14.	Explain the various Pitch Errors in Screw Threads.

15.	Explain about the Laser Equipment for Alignment Testing.

PART – C (5 x 15 = 75 MARKS)

16.	Explain in detail, the different types of errors in measurement and their causes.

(OR)

17.	Define Calibration. Explain in detail the Calibration Service Programme.

18.	State the types of Comparators. Explain the Mechanical Comparator in detail.

(OR)

19.	Explain with neat sketch, the principle and construction of an Autocollimator.

20.	Explain in detail, the methods of measuring Surface Finish.

(OR)

21.	Explain the construction and working of the Tool Makers Microscope.

22.	Draw and explain the measurement of effective diameter of a screw thread using 3 – wire method.

(OR)

23.	Explain in detail, the various Gear Errors.

[P.T.O]

24.	a.	Explain about the performance evaluation of CMM.								(7)

b.	List out the applications of CMM.												(8)

(OR)

25.	Explain in detail, the principle and working of LASER Interferometer.